APPENDIX A

A.1 Literature Review of Risk Factors for Foot Ulcers

- “The sole under the metatarsal heads functions as a shock absorber during walking and running. The mechanical properties of the sole provide the primary defence against the development of foot ulceration”. (Wang et al. 1999) - It was found that “The plantar soft tissues of the elderly diabetic patients were significantly stiffer and thinner when with healthy young subjects” (Zheng et al. 2000) - “Studies have suggested that vertical force is the predominant factor in stimulating calluses. together with prolonged and repetitive duration of increased pressure” (Potter, 1999). - “It is important to know how a transducer, which may have a fairly large pressure sensing area, summates forces that are varying from point to point across its surface.” Hutton et al. (cited in Klenerman, 1982) - “A few numbers of pressure sensors under the sole of the foot is the problem of interfering the pressure elsewhere on the foot. Unless a very large number of small transducers are used, there is no possibility of integrating pressure values over a specific are to derive the total force transmitted”. Hutton et al. (Cited in Klenerman, 1982). - Young et al (cited in Cavanagh, 2001) have shown that plantar callus can significantly increase plantar pressure during walking. - “More patients are at risk of neuropathic than ischaemic ulceration” (Klenerman and Geary, 1991) - “Claw toes are no more common in diabetic people than in non-diabetic people, although diabetes is more common in older people, as are claw toes” (Thomas, 1982) - “Abnormally high plantar pressures are common in diabetic patients, particularly on the metatarsal heads, and they change the patterns of weight-bearing” (Boulton et al. 1985). - “High pressure alone is not a direct cause of ulceration, but ulcers almost invariably occur at sites where abnormally high pressures can be demonstrated” (Masson et al. 1989; Cavanag & Ulbrecht, 1991; and Fernando et al. 1991) and “The presence of sensory neuropathy and elevated pressure are related to the development of foot ulcers” (Sanmarco and Stephens, 1991). - “Body weight and activity level will increase the force that the foot must transmit, and this may increase pressures, especially if there is an underlying bony prominence or foot imbalance” (Sanmarco & Stephens, 1991). - “Increases in shear forces and gait abnormalities caused by deformities or badly fitting shoes become factors in skin breakdown” (Sanmarco & Stephens, 1991), and tight, narrow shoes can produce high pressures and increase chances of ulceration (Jesse, 1995). - “Pressure measurements and biomechanical assessments are now well established as important techniques for identifying feet that are at risk of ulceration and re-ulceration” (Cavanagh, 2000). - “Important changes in the distribution and level of pressures under diabetic neuropathic feet occur during a relatively short time” (Boulton et al. 1986). - “ The sole under the metatarsal heads functions as a shock absorber during walking and running. The mechanical properties of the sole provide the primary defence against the development of foot ulceration”(Wang et al, 1999). - “The sole under the first metatarsal head had the greatest values for compressibility index and elastic modulus (55.9% and 1.39kg/cm2), and the sole under the third metatarsal head had the smallest values (50.8% and 1.23 kg/cm2).” (Wang et al, 1999). - “ The sole under the metatarsal head is thinner than that of the heel. and its thickness decreases progressively from the first to the fifth metatarsal head.” - It was found that “ Significantly higher effective shear and elastic muduli were measured for the diabetic plantar tissue compared with that of normal. For the diabetic patients local stiffness in proximately of the 1st metatarsal head was substantially larger than in other sites, and this could be associated with initial tissue ulceration at this location.” ( Gefen et al 2001) - It was found that “The plantar soft tissues of the elderly diabetic patients were significantly stiffer and thinner when with healthy young subjects.” (`Zheng et al. 2000). - “A reliable tissue evaluation procedure and proper footwear are extremely important to protect diabetics from tissue ulceration and amputation.” (`Zheng et al, 2000). - “The accurate measure of the thickness of the protective plantar tissues, particularly with the foot loaded, there fore provides potentially valuable data in diagnosing, assessing and planning treatment for diabetic patients”.( Bygrave & Betts, 1992). - It was found in a study of a group of normal subjects that a gradually increasing depth of tissue when moving from the fifth to the first metatarsal head. ( Bygrave & Betts, 1992). - “Studies have suggested that vertical force is the predominant factor in stimulating calluses. together with prolonged and repetitive duration of increased pressure” ( Potter, 1999). In the same studies, there are several methods suggested for prevention and treatment of foot ulceration: - Proper diabetic foot clinics should be available, providing good facilities for diagnosis (such as laboratories for gait analysis, clinical analysis and x-rays), treatment, management of diabetes, and education in foot care. - The management of diabetic feet must be the responsibility of a multidisciplinary team, which includes podiatrists, orthopaedic surgeons, orthotics specialists, nursing staff, endocrinologists, diabetologists, and nutritionists. It is important that every member of this team is properly qualified, and that there is proper communication within the team in order to monitor the patient throughout the illness. - Proper protective footwear, ideally customised, must be available - Observation of each patient should be carried out on a regular basis, in order to ensure that the treatment is still appropriate, and to adjust treatment and modify footwear if necessary. - Pressure measurements can be used to evaluate the effectiveness of footwear intervention in patients with and without ulcers. - There should be proper databases of clinical information, which are kept up to date, so that everyone involved in a patient’s treatment has ready access to relevant information. The information should be in a form that can be understood by the whole team.