LITERATURE REVIEW

INTER COMPARISON OF MEDICAL IMAGE SEGMENTATION ALGORITHMS.

Submitted to: Dr. William Sandham.

By: Amit. N. Pujari.

MRI INTRODUCTION:

Magnetic resonance imaging was developed from knowledge gained in the study of nuclear magnetic resonance. The original name for the medical technology is nuclear magnetic resonance imaging (NMRI), but the word nuclear is almost universally dropped. This is done to avoid the negative connotations of the word nuclear, and to prevent patients from associating the examination with radiation exposure, which is not one of the safety concerns for MRI.

### Technique

Medical MRI most frequently relies on the relaxation properties of excited hydrogen nuclei in water. When the object to be imaged is placed in a powerful, uniform magnetic field, the spins of the atomic nuclei with non-zero spin numbers within the tissue all align in one of two opposite directions: parallel to the magnetic field or antiparallel. Common magnetic field strengths range from 0.3 to 3 teslas, although research instruments range as high as 20 teslas, and commercial suppliers are investing in 7 tesla platforms.

An excess of only one in a million nuclei align themselves with the magnetic field, since the thermal energy far exceeds the difference between the parallel and antiparallel states. Yet, the vast quantity of nuclei in a small volume sum to produce a detectable change in field. Most basic explanations of NMR and MRI will say that the nuclei align parallel or anti-parallel with the static magnetic field; however, because of quantum mechanical reasons beyond the scope of this article, the individual nuclei are actually set off at an angle from the direction of the static magnetic field, although the bulk collection of nuclei can be partitioned into a set whose sum spin are aligned parallel and a set whose sum spin are anti-parallel.

The magnetic dipole moment of the nuclei then precesses around the axial field. While the proportion is nearly equal, slightly more are oriented at the low energy angle. The frequency with which the dipole moments precess is called the Larmor frequency. The tissue is then briefly exposed to pulses of electromagnetic energy (RF pulse) in a plane perpendicular to the magnetic field, causing some of the magnetically aligned hydrogen nuclei to assume a temporary non-aligned high-energy state. The frequency of the pulses is governed by the Larmor equation.

In order to selectively image different voxels (picture elements) of the material in question, orthogonal magnetic gradients are applied. Although it is relatively common to apply gradients in the principal axes of a patient (so that the patient is imaged in x, y, and z from head to toe), MRI allows completely flexible orientations for images. All spatial encoding is obtained by applying magnetic field gradients which encode position within the phase of the signal. In 1 dimension, a linear phase with respect to position can be obtained by collecting data in the presence of a magnetic field gradient. In 3 dimensions, a plane can defined by "slice selection", in which an RF pulse of defined bandwidth is applied in the presence of a magnetic field gradient in order to reduce spatial encoding to 2 dimensions. Spatial encoding can then be applied in 2D after slice selection, or in 3D without slice selection. In either case, a 2D or 3D matrix of spatially-encoded phases is acquired, and these data represent the spatial frequencies of the image object. Images can be created from the acquired data using the Discrete Fourier Transform (DFT).

In order to understand MRI contrast, it is important to have some understanding of the time constants involved in relaxation processes that establish equilibrium following RF excitation. As the high-energy nuclei relax and realign, they emit energy at rates which are recorded to provide information about their environment. The realignment of nuclear spins with the magnetic field is termed longitudinal relaxation and the time (typically about 1 sec) required for a certain percentage of the tissue nuclei to realign is termed "Time 1" or T1. T2-weighted imaging relies upon local dephasing of spins following the application of the transverse energy pulse; the transverse relaxation time (typically < 100 ms for tissue) is termed "Time 2" or T2. A subtle but important variant of the T2 technique is called T2* imaging. T2 imaging employs a spin echo technique, in which spins are refocused to compensate for local magnetic field inhomogeneities. T2* imaging is performed without refocusing. This sacrifices some image integrity in order to provide additional sensitivity to relaxation processes that cause incoherence of transverse magnetization. Applications of T2* imaging include function MRI (fMRI) or evaluation of baseline perfusion (CBF and CBV) using injected agents as described above; in these cases, there is an inherent trade-off between image quality and detection sensitivity.

Image contrast is created by using a selection of image acquisition parameters that weights signal by T1, T2 or T2*, or no relaxation time ("proton-density images"). In the brain, T1-weighting causes fiber tracts (nerve connections) to appear white, congregations of neurons to appear gray, and cerebrospinal fluid to appear dark. The contrast of "white matter," "gray matter'" and "cerebrospinal fluid" is reversed using T2 or T2* imaging, whereas proton-weighted imaging provides little contrast in normal subjects. Additionally, functional information (CBF, CBV, blood oxygenation) can be encoded within T1, T2, or T2*; see functional MRI (fMRI) and the section below.

Typical medical resolution is about 1 mm3, while research models can exceed 1 µm3.

Contrast-enhancement:

Both T1- and T2-weighted images are acquired for most medical examinations. However, these 2 sets of images are not always sufficient to adequately show anatomy or pathology. One option is to use a more sophisticated image acquisition technique - e.g. fat suppression, chemical-shift imaging. The other is to administer a contrast agent to delineate areas of interest.

## Application :

In clinical practice, MRI is used to distinguish pathologic tissue (such as a brain tumor) from normal tissue. One of the advantages of an MRI scan is that, according to current medical knowledge, it is harmless to the patient. It utilizes strong magnetic fields and non-ionizing radiation in the radio frequency range. Compare this to CT scans and traditional X-rays which involve doses of ionizing radiation and may increase the chance of malignancy, especially in a fetus.

While CT provides good spatial resolution (the ability to distinguish two structures an arbitrarily small distance from each other as separate), MRI provides comparable resolution with far better contrast resolution (the ability to distinguish the differences between two arbitrarily similar but not identical tissues). The basis of this ability is the complex library of pulse sequences that the modern medical MRI scanner includes, each of which is optimized to provide image contrast based on the chemical sensitivity of MRI.

For example, with particular values of the echo time (TE) and the repetition time (TR), which are basic parameters of image acquisition, a sequence will take on the property of T2-weighting. On a T2-weighted scan, fat-, water- and fluid-containing tissues are bright (most modern T2 sequences are actually fast T2 sequences). Damaged tissue tends to develop edema, which makes a T2-weighted sequence sensitive for pathology, and generally able to distinguish pathologic tissue from normal tissue. With the addition of an additional radio frequency pulse and additional manipulation of the magnetic gradients, a T2-weighted sequence can be converted to a FLAIR (Fluid Light Attenuation Inversion Recovery) sequence, in which free water is now dark, but edematous tissues remain bright. This sequence in particular is currently the most sensitive way to evaluate the brain for demyelinating diseases, such as multiple sclerosis.

The typical MRI examination consists of 5-20 sequences, each of which are chosen to provide a particular type of information about the subject tissues. This information is then synthesized by the interpreting physician.

Brain and MRI:

As discussed above MRI  has very good contrast resolution, therefore it is very good imaging technique for complex structure like brain.

Concentration of this project is on brain MRI images.

Brain can be devided into Cerebro spinal fluid (CSF), Gray matter (GM) and White matter (WM); and ufcourse we have to separate non brain region from this.

Image segmentation:

Image segmentation is a procedure that partitions an image in a one or more regions of interest.The extraction procedure responds to some criateria such as grey level, colour or texture and it is straightly related to the concrete application.

Image segmentation used to distinguish different tissues (in this context brain tissues)  in the medical image , so that some specific measures can be automatically done.

Image segmentation is an important field in image processing, and many techniques have been putforth throught years and yet new methods are being explored.

Segmentation methods:

Thresolding: Thresholding is the simplest method of image segmentation. Individual pixels in a grayscale image are marked as 'object' pixels if their value is greater than some threshold value (assuming an object to be brighter than the background) and as 'background' pixels otherwise. Typically, an object pixel is given a value of '1' while a background pixel is given a value of '0'.

The key parameter in threholding is obviously the choice of the threshold. Several different methods for choosing a threshold exist. The simplest method would be to choose the mean or median value, the rationale being that if the object pixels are brighter than the background, they should also be brighter than the average. In a noiseless image with uniform background and object values, the mean or median will work beautifully as the threshold, however generally speaking, this will not be the case. A more sophisticated approach might be to create a histogram of the image pixel intensities and use the valley point as the threshold. The histogram approach assumes that there is some average value for the background and object pixels, but that the actual pixel values have some variation around these average values. However, computationally this is not as simple as we'd like, and many image histograms do not have clearly defined valley points. Ideally we're looking for a method for choosing the threshold which is simple, does not require too much prior knowledge of the image, and works well for noisy images. A good such approach is an iterative metod as follows:

An initial threshold (T) is chosen, this can be done randomly or according to any other method desired.

The image is segmented into object and background pixels as described above, creating two sets:

G1 = {f(m,n):f(m,n)>T} (object pixels)

G2 = {f(m,n):f(m,n)T} (background pixels) (note, f(m,n) is the value of the pixel located in the mth column, nth row)

The average of each set is computed.

m1 = average value of G1

m2 = average value of G2

A new threshold is created that is the average of m1 and m2

T' = (m1 + m2)/2

Go back to step two, now using the new threshold computed in step 4, keep repeating until the new threshold matches the one before it (i.e. until convergence has been reached).

Another approach is to calculate the new threshold in step 5 using the weighted average of m1 and m2: T' = (||G1||*m1 + ||G2||*m2)/(||G1||+||G2||), where ||Gn|| is the number of pixels in Gn. This approach often gives a more accurate result.

This iterative algorithm is a special one-dimensional case of the k-means algorithm, which has been proven to converge at a local minimum - meaning that a different initial threshold may result in a different final result.

2. Region growing:

Region growing can be seen as a combination of thresholding and pixel connectivity. This method exploits the fact that close pixels probably have similar gray level values. Intially we should know how many regions are present in the image and some pixels belonging to them, called seeds. Starting from these seeds, the region growing algorithm looks for pixels which are connected to them, following normally an intensity and a connectivity citerion. This way, the region grows.

3. Clustering:   Data clustering is a common technique for statistical data analysis, which is used in many fields, including machine learning, data mining, pattern recognition, image analysis and bioinformatics. Clustering is the classification of similar objects into different groups, or more precisely, the partitioning of a data set into subsets (clusters), so that the data in each subset (ideally) share some common trait - often proximity according to some defined distance measure.

4. There are some other methods like edge based segmentation.

Edge based segmentation is not a stand alone technique for medical image segmentation, as it depends on methods in other categories to avoid the disadvantages of edge detection itself.

Segmentation algoritms:

There are number of trusted and efficient algorithms each having its own advantages.

To mention few are: 1. Otsu’s algorithm, 2. Kittler’s algorithm, 3. C-means, 4. K-means.

Fuzzy logic: Fuzzy sets have way of representing data by mapping each number in the set into the interval. For an example, the image is up of a number of pixels having gray values. The image can be regarded as a fuzzy set, and each pixel can be regarded as having a value that represents its fuzzy membership to the set.

There are algorithms based on fuzzy logic, like fuzzy c- means, fuzzy clustering.

Fuzzy c-means clustering :

In fuzzy clustering, each point has a degree of belonging to clusters, as in fuzzy logic, rather than belonging completely to just one cluster. Thus, points on the edge of a cluster, may be in the cluster to a lesser degree than points in the center of cluster. For each point x we have a coefficient giving the degree of being in the kth cluster uk(x). Usually, the sum of those coefficients is defined to be 1, so that uk(x) denotes a probability of belonging to a certain cluster:

With fuzzy c-means, the centroid of a cluster is the mean of all points, weighted by their degree of belonging to the cluster:

The degree of belonging is related to the inverse of the distance to the cluster

then the coefficients are normalized and fuzzyfied with a real parameter m > 1 so that their sum is 1. So

For m equal to 2, this is equivalent to normalising the coefficient linearly to make their sum 1. When m is close to 1, then cluster center closest to the point is given much more weight than the others, and the algorithm is similar to k-means.

The fuzzy c-means algorithm is very similar to the k-means algorithm:

Choose a number of clusters.

Assign randomly to each point coefficients for being in the clusters.

Repeat until the algorithm has converged (that is, the coefficients' change between two iterations is no more than ε, the given sensitivity threshold) :

Compute the centroid for each cluster, using the formula above.

For each point, compute its coefficients of being in the clusters, using the formula above.

The algorithm minimizes intra-cluster variance as well, but has the same problems as k-means, the minimum is a local minimum, and the results depend on the initial choice of weights.

K-means clustering :

The K-means algorithm assigns each point to the cluster whose center (also called centroid) is nearest. The center is the average of all the points in the cluster — that is, its coordinates are the arithmetic mean for each dimension separately over all the points in the cluster.

Example: The data set has three dimensions and the cluster has two points: X = (x1, x2, x3) and Y = (y1, y2, y3). Then the centroid Z becomes Z = (z1, z2, z3), where z1 = (x1 + y1)/2 and z2 = (x2 + y2)/2 and z3 = (x3 + y3)/2.

The algorithm is roughly (J. MacQueen, 1967):

Randomly generate k clusters and determine the cluster centers, or directly generate k seed points as cluster centers.

Assign each point to the nearest cluster center.

Recompute the new cluster centers.

Repeat until some convergence criterion is met (usually that the assignment hasn't changed).

The main advantages of this algorithm are its simplicity and speed which allows it to run on large datasets. Its disadvantage is that it does not yield the same result with each run, since the resulting clusters depend on the initial random assignments. It maximizes inter-cluster (or minimizes intra-cluster) variance, but does not ensure that the result has a global minimum of variance.

Inter comparison of different algorithms:

Objective of this project is to compare existing algorithms and to develop better segmentatin algoritm for brain MRI images.

Software:

For analysis, design and development MATLAB software will be used alongwith its siutable packages like signal processing, wavlet transform.

Synthetic data:

Synthetic data will be taken from

http://www.bic.mni.mcgill.ca/brainweb/

Clinical data:

Clinical data will be obtained from contact and consent from Southern general hospital, Glasow, UK.

Reference:

1. A.K. Jain, Fundamentals of Digital Image Processing, Prentice Hall, 1989.

2. R.C. Gonzalez and P. Wintz, Digital Image Processing, Addison-

Wesley1987.

3. W.K. Pratt, Digital Image Processing, Wiley, New York, 1978

4. Shan Shen (2004) “ MRI brain tumor classification using image processing

and data mining”, University of strathclyde glasgow.

5. Pablo Alcoriza Vento “MRI data processing”, University of strathclyde

glasgow.

6. Warren Halberstadt “pattern recognition in the detection of tuberculosis

meningitis”, University of capetown.

7. http://www.bic.mni.mcgill.ca/brainweb/

8. S. Shen, W.A. Sandham, M.H. Granat, et, al, “Fuzzy clustering based

applications to medical image segmentation”, Proc of annual international

conference of the IEEE engineering in medicine and biology society, 17-21

Sept 2003, Cancun, Mexico.

9. S. Shen, W.A. Sandham, M.H. Granat, “Preporcessing and segmentataion

of brain magnetic resonanace images”, Proc ITAB 2003, Birmimgham UK.

10. Shan Shen, William Sandham, et, al, “MRI fuzzy segmentation of brain

tissue using neighborhood attraction with neural network”, IEEE

trasaction on information technology in biomedicine, vol9, no.3,

september 2005.

11. H. Suzuki and J. Toriwaki, “ Automatic segmentatio of the head MRI

images by knowledge guided thresholding”, Computerised medical

imaging and graphics, vol, 15, no 4, pp. 223-240, 1991.

12. L.Lemieux, et, al, “Fast , accurate and reproducible automatic

segmentation of the brain in T1- wighted volume MRI data”, magnetic

resonance in medicine, vol 42, pp 127-135, 1999.

13.  W.M. Wells, III, et, al, “Adaptive segmentation of MRI data” IEEE trans.

Med. Imag. Vol.15, no.4, pp. 740-750,1993.

14.Chulin Li, et, al,”Knowledge based classification and tissue labeling of MR

images of human brain”, IEEE transaction. Med ima. Vol.12,no.4,pp 740-

750,1993.

15.J.C.Bezek, Pattern recognition with fuzzy object function algorithms,

Plenump press, 1981.

16.M.N. Ahmed, et, al, “Bias field estimation and adaptive segmentation of

MRI data using a modified fuzzy C-means algorithm” Comuter vision and

pattern recognition, vol.1, pp 1250-1255, 1999.

17. D.L. Pham and J.L. Prince, “Adapative fuzzy sgmentation of magnetic

resonance images”, IEEE trans. Med imag, vol. 18, no.9,pp 737-752,

1999.

18.R.N. Dave and R.Krishnapuram, “Robust clustering methods: A unified

view”, IEEE transaction. Fuzzy sys, vol. 5, pp 270-293, 1997.

19.L.G. Nyul, J.K. Udupa, “On standerdising the MR image intensity scales”,

Magnetic resonance imaging, vol 42, pp 1072-1081, 1999.

20. A.F. Siegel, C.J. Morgan, Statistics and data analysis-An introduction, 2nd

ed, john wiley,and sons, Inc.1996.

21.B.S. Everitt, “Statistical methods for medical investigations”,  Oxford

university press, 1999.