Appendix 2B1

Matlab Program

clear % clears matlabs memory

close all; %closes all matlab programs open

format short

%open data file as read only

fid = fopen('c:\Old Program Files\BM\BM4CH\davI1.dat','r') %open  file

%open file

raw_data = fread(fid); %raw_data (Datalogger data)

status = fclose(fid) %close file

freq = input('Specify frequency sampling rate:   ') %frequency

sampling   rate

num_of_chan = input('Enter number of channels:   ') %number of   channels

length_of_data = length(raw_data);

TIME= length_of_data/freq; %total program running time

length_of_data1 = length_of_data/num_of_chan; % number of samples   per channel

%The graph Program

time = (TIME/length_of_data)*(1:length_of_data1); % defines time  per channel

time1 = max(size(time))-1; %velocity array

fprintf('The frequency sampling rate is %g\n.', freq);

fprintf('The number of channels used is %g\n.', num_of_chan);

if num_of_chan > 4

fprintf('An error has occurred, enter integer between 1 and 3')

end

if num_of_chan == 4

Gonio_title = input('Enter Goniometer Name for Channel 1: ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle (not time), %c = intercept (constant)= angle1, m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111;

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s3b','S3b'}

angle1 = 128.51;

angle2 = 1.2943;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s5b','S5b'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'s6b','S6b'}

angle1 = 126.86;

angle2 = 1.1368;

case{'s7','S7'}

angle1 = 129.7;

angle2 = 1.2307;

case{'s8','S8'}

angle1 = 128.22;

angle2 = 1.2238;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

% Program designed to remove 1 bit amplitude noise from the ELECTROGONIOMETRIC signal

chan1b = raw_data(1:4:length_of_data);

for i = 1:max(size(chan1b))-3

if (abs(chan1b(i) - chan1b(i+1)) == 1)

if (abs(chan1b(i+1) - chan1b(i+2)) == 1)

if chan1b(i)== chan1b(i+2)

chan1b(i+1) = chan1b(i);

end

else

if (abs(chan1b(i+1) - chan1b(i+2)) == 0)

if (abs(chan1b(i+2) - chan1b(i+3)) == 1)

if chan1b(i)== chan1b(i+3)

chan1b(i+1) = chan1b(i);

chan1b(i+2) = chan1b(i);

end

end

end

end

end

end

%chan1 = (raw_data(1:4:length_of_data) - angle1)/angle2; %Joint angles for chan1

%this formular is useful for comparing the noisy signal with the 'filtered' one.

chan1 = (chan1b - angle1)/angle2;%Joint angles for chan1

Gonio_title = input('Enter Goniometer Name for Channel 2: ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle, c = intercept (constant)= angle1 %m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111;

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s3b','S3b'}

angle1 = 128.51;

angle2 = 1.2943;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s5b','S5b'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'s6b','S6b'}

angle1 = 126.86;

angle2 = 1.1368;

case{'s7','S7'}

angle1 = 129.7;

angle2 = 1.2307;

case{'s8','S8'}

angle1 = 128.22;

angle2 = 1.2238;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

% Program designed to remove 1 bit amplitude noise from the ELECTROGONIOMETRIC signal

chan2b = raw_data(2:4:length_of_data);

for i = 1:max(size(chan2b))-3

if (abs(chan2b(i) - chan2b(i+1)) == 1)

if (abs(chan2b(i+1) - chan2b(i+2)) == 1)

if chan2b(i)== chan2b(i+2)

chan2b(i+1) = chan2b(i);

end

else

if (abs(chan2b(i+1) - chan2b(i+2)) == 0)

if (abs(chan2b(i+2) - chan2b(i+3)) == 1)

if chan2b(i)== chan2b(i+3)

chan2b(i+1) = chan2b(i);

chan2b(i+2) = chan2b(i);

end

end

end

end

end

end

% chan2 = (raw_data(2:4:length_of_data) - angle1)/angle2; %Joint   angles for chan2

chan2 = (chan2b - angle1)/angle2;

Gonio_title = input('Enter Goniometer Name for Channel 3: ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle,

c = intercept (constant)= angle1  m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111;

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s3b','S3b'}

angle1 = 128.51;

angle2 = 1.2943;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s5b','S5b'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'s6b','S6b'}

angle1 = 126.86;

angle2 = 1.1368;

case{'s7','S7'}

angle1 = 129.7;

angle2 = 1.2307;

case{'s8','S8'}

angle1 = 128.22;

angle2 = 1.2238;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

% Program designed to remove 1 bit amplitude noise from the ELECTROGONIOMETRIC signal

chan3b = raw_data(3:4:length_of_data);

for i = 1:max(size(chan3b))-3

if (abs(chan3b(i) - chan3b(i+1)) == 1)

if (abs(chan3b(i+1) - chan3b(i+2)) == 1)

if chan3b(i)== chan3b(i+2)

chan3b(i+1) = chan3b(i);

end

else

if (abs(chan3b(i+1) - chan3b(i+2)) == 0)

if (abs(chan3b(i+2) - chan3b(i+3)) == 1)

if chan3b(i)== chan3b(i+3)

chan3b(i+1) = chan3b(i);

chan3b(i+2) = chan3b(i);

end

end

end

end

end

end

% chan3 = (raw_data(3:4:length_of_data) - angle1)/angle2; %Joint angles for chan3

chan3 = (chan3b - angle1)/angle2;

Gonio_title = input('Enter Goniometer Name for Channel 4: ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle, c = intercept (constant)= angle1  m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111;

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s3b','S3b'}

angle1 = 128.51;

angle2 = 1.2943;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s5b','S5b'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'s6b','S6b'}

angle1 = 126.86;

angle2 = 1.1368;

case{'s7','S7'}

angle1 = 129.7;

angle2 = 1.2307;

case{'s8','S8'}

angle1 = 128.22;

angle2 = 1.2238;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

% Program designed to remove 1 bit amplitude noise from the ELECTROGONIOMETRIC signal

chan4b = raw_data(4:4:length_of_data);

for i = 1:max(size(chan4b))-3

if (abs(chan4b(i) - chan4b(i+1)) == 1)

if (abs(chan4b(i+1) - chan4b(i+2)) == 1)

if chan4b(i)== chan4b(i+2)

chan4b(i+1) = chan4b(i);

end

else

if (abs(chan4b(i+1) - chan4b(i+2)) == 0)

if (abs(chan4b(i+2) - chan4b(i+3)) == 1)

if chan4b(i)== chan4b(i+3)

chan4b(i+1) = chan4b(i);

chan4b(i+2) = chan4b(i);

end

end

end

end

end

end

% chan4 = (raw_data(4:4:length_of_data) - angle1)/angle2; %Joint angles for chan4

chan4 = (chan4b - angle1)/angle2;

%*******************************************************************

%Average filter

x=0;

z=0;

chan1x=[x;chan1(1:(max(size(chan1))-1))]; %f(n-1)

chan1y=[chan1(1:max(size(chan1)))]; %f(n)              chan1z=[chan1(2:max(size(chan1)));z]; %f(n+1)

chan1f=[chan1x+chan1y+chan1z]/3; %filtered data

chan2x=[x;chan2(1:(max(size(chan2))-1))]; %f(n-1)

chan2y=[chan2(1:max(size(chan2)))]; %f(n)

chan2z=[chan2(2:max(size(chan2)));z]; %f(n+1)

chan2f=[chan2x+chan2y+chan2z]/3; %filtered data

chan3x=[x;chan3(1:(max(size(chan3))-1))]; %f(n-1)

chan3y=[chan3(1:max(size(chan3)))]; %f(n)

chan3z=[chan3(2:max(size(chan3)));z]; %f(n+1)

chan3f=[chan3x+chan3y+chan3z]/3; %filtered data

chan4x=[x;chan4(1:(max(size(chan4))-1))]; %f(n-1)

chan4y=[chan4(1:max(size(chan4)))]; %f(n)

chan4z=[chan4(2:max(size(chan4)));z]; %f(n+1)

chan4f=[chan4x+chan4y+chan4z]/3; %filtered data

%*******************************************************************

figure;

plot(time,chan1f,'r');

hold on

plot(time,chan2f,'g');

hold on

plot(time,chan3f,'b');

hold on

plot(time,chan4f,'k')

grid on;

zoom on;

title('Graph of Joint Angle Against Time');

ylabel('Joint Angle (Degrees)');

xlabel('Time (Seconds)');

legend('chan1f','chan2f','chan3f','chan4f')

%*******************************************************************

% velocity (first derivative)& acceleration calculations (second derivative)

% formula:

% vel=(1/12*D)*(f(n-2)-8*f(n-1)+8*f(n+1)-f(n+2))

% where D=interval (time) b/w two adjacent points, f(n)= data points

x=0; y=0; z=0; D=time(2)-time(1);

velw=[x;y;chan1f(1:(max(size(chan1f))-2))];

velx=-8*[y;chan1f(1:(max(size(chan1f))-1))];

vely=8*[chan1f(2:max(size(chan1f)));x];

velz=-[chan1f(3:max(size(chan1f)));x;z];

velf=1/(12*D)*[velw+velx+vely+velz];

vel2w=[x;y;chan2f(1:(max(size(chan2f))-2))];

vel2x=-8*[y;chan2f(1:(max(size(chan2f))-1))];

vel2y=8*[chan2f(2:max(size(chan2f)));x];

vel2z=-[chan2f(3:max(size(chan2f)));x;z];

vel2f=1/(12*D)*[vel2w+vel2x+vel2y+vel2z];

vel3w=[x;y;chan3f(1:(max(size(chan3f))-2))];

vel3x=-8*[y;chan3f(1:(max(size(chan3f))-1))];

vel3y=8*[chan3f(2:max(size(chan3f)));x];

vel3z=-[chan3f(3:max(size(chan3f)));x;z];

vel3f=1/(12*D)*[vel3w+vel3x+vel3y+vel3z];

vel4w=[x;y;chan4f(1:(max(size(chan4f))-2))];

vel4x=-8*[y;chan4f(1:(max(size(chan4f))-1))];

vel4y=8*[chan4f(2:max(size(chan4f)));x];

vel4z=-[chan4f(3:max(size(chan4f)));x;z];

vel4f=1/(12*D)*[vel4w+vel4x+vel4y+vel4z];

%******************************************************************

figure;

subplot(2,1,1);

plot(time,chan1f,'r');

grid on;

zoom on;

ylabel('Joint Angle (Degrees)');

title('Graph of Channel 1');

subplot(2,1,2);

plot(time,velf,'b');

ylabel('Velocity');

grid on;

zoom on;

xlabel('Time (Seconds)');

figure;

subplot(2,1,1);

plot(time,chan2f,'b');

grid on;

zoom on;

ylabel('Joint Angle (Degrees)');

title('Graph of Channel 2');

subplot(2,1,2);

plot(time,vel2f,'g');

ylabel('Velocity');

grid on;

zoom on;

xlabel('Time (Seconds)');

figure;

subplot(2,1,1);

plot(time,chan3f,'g');

grid on;

zoom on

ylabel('Joint Angle (Degrees)');

title('Graph of Channel 3');

subplot(2,1,2);

plot(time,vel3f,'b');

ylabel('Velocity');

grid on;

zoom on

xlabel('Time (Seconds)');

figure;

subplot(2,1,1);

plot(time,chan4f,'k');

grid on;

zoom on

ylabel('Joint Angle (Degrees)');

title('Graph of Channel 4');

subplot(2,1,2);

plot(time,vel4f,'b');

ylabel('Velocity');

grid on;

zoom on

xlabel('Time (Seconds)');

%The Frequency Program (Number of Occurrences)

chan1a=round(chan1f);

countp=zeros(1,120);

countn=zeros(1,150);

countz=0;

for i = 1:max(size(chan1a))

if chan1a(i) < 0

countn(abs(chan1a(i)))=countn(abs(chan1a(i)))+1;

end

if chan1a(i) > 0

countp(chan1a(i))=countp(chan1a(i))+1;

end

if chan1a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(90:-1:1);

h=[cn countz countp];

hx=-90:1:120;

figure;

plot(hx,h);

grid on;

zoom on;

bar(hx,h);

title('Graph Showing the Frequency of Occurrences of the Joint Angles for Channel 1');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

chan2a=round(chan2f);

countp=zeros(1,120);

countn=zeros(1,150);

countz=0;

for i = 1:max(size(chan2a))

if chan2a(i) < 0

countn(abs(chan2a(i)))=countn(abs(chan2a(i)))+1;

end

if chan2a(i) > 0

countp(chan2a(i))=countp(chan2a(i))+1;

end

if chan2a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(90:-1:1);

h=[cn countz countp];

hx=-90:1:120;

figure;

plot(hx,h);

grid on;

zoom on;

bar(hx,h);

title('Graph Showing the Frequency of Occurrences of the Joint Angles for Channel 2');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

chan3a=round(chan3f);

countp=zeros(1,120);

countn=zeros(1,150);

countz=0;

for i = 1:max(size(chan3a))

if chan3a(i) < 0

countn(abs(chan3a(i)))=countn(abs(chan3a(i)))+1;

end

if chan3a(i) > 0

countp(chan3a(i))=countp(chan3a(i))+1;

end

if chan3a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(90:-1:1);

h=[cn countz countp];

hx=-90:1:120;

figure;

plot(hx,h);

grid on;

zoom on;

bar(hx,h);

title('Graph Showing the Frequency of Occurrences of the Joint Angles for Channel 3');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

chan4a=round(chan4f);

countp=zeros(1,120);

countn=zeros(1,150);

countz=0;

for i = 1:max(size(chan4a))

if chan4a(i) < 0

countn(abs(chan4a(i)))=countn(abs(chan4a(i)))+1;

end

if chan4a(i) > 0

countp(chan4a(i))=countp(chan4a(i))+1;

end

if chan4a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(120:-1:1);

h=[cn countz countp];

hx=-120:1:120;

figure;

plot(hx,h);

grid on;

bar(hx,h);

zoom on;

title('Graph Showing the Frequency of Occurrences of the Joint Angles for Channel 4');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

% Statistics

mean1 = mean(chan1f);

fprintf('The mean for channel 1 is %g\n',mean1);

mean2 = mean(chan2f);

fprintf('The mean for channel 2 is %g\n',mean2);

mean3 = mean(chan3f);

fprintf('The mean for channel 3 is %g\n',mean3);

mean4 = mean(chan4f);

fprintf('The mean for channel 4 is %g\n',mean4);

min1 = min(chan1f);

fprintf('The minimum value for channel 1 is %g\n',min1);

min2 = min(chan2f);

fprintf('The minimum value for channel 2 is %g\n',min2);

min3 = min(chan3f);

fprintf('The minimum value for channel 3 is %g\n',min3);

min4 = min(chan4f);

fprintf('The minimum value for channel 4 is %g\n',min4);

max1 = max(chan1f);

fprintf('The maximum value for channel 1 is %g\n',max1);

max2 = max(chan2f);

fprintf('The maximum value for channel 2 is %g\n',max2);

max3 = max(chan3f);

fprintf('The maximum value for channel 3 is %g\n',max3);

max4 = max(chan4f);

fprintf('The maximum value for channel 4 is %g\n',max4);

range1 = max1 - min1 ;

fprintf('The range for channel 1 is %g\n',range1);

range2 = max2 - min2;

fprintf('The range for channel 2 is %g\n',range2);

range3 = max3 - min3;

fprintf('The range for channel 3 is %g\n',range3);

range4 = max4 - min4;

fprintf('The range for channel 4 is %g\n',range4);

elseif num_of_chan == 3

Gonio_title = input('Enter Goniometer Name for Channel 1:  ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle, c = intercept (constant)= angle1 m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111;

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

chan1 = (raw_data(1:3:length_of_data) - angle1)/angle2; %Joint angles for chan1

Gonio_title = input('Enter Goniometer Name for Channel 2:  ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle, c = intercept (constant)= angle1, %m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111;

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

chan2 = (raw_data(2:3:length_of_data) - angle1)/angle2; %Joint angles for chan2

Gonio_title = input('Enter Goniometer Name for Channel 3:  ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle, c = intercept (constant)= angle1, %m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111;

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

chan3 = (raw_data(3:3:length_of_data) - angle1)/angle2; %Joint angles for chan3

%*********************************************************************

x=0;

z=0;

chan1x=[x;chan1(1:(max(size(chan1))-1))]; %f(n-1)               chan1y=[chan1(1:max(size(chan1)))]; %f(n)             chan1z=[chan1(2:max(size(chan1)));z]; %f(n+1)

chan1f=[chan1x+chan1y+chan1z]/3; %filtered data

chan2x=[x;chan2(1:(max(size(chan2))-1))]; %f(n-1)

chan2y=[chan2(1:max(size(chan2)))]; %f(n)

chan2z=[chan2(2:max(size(chan2)));z]; %f(n+1)

chan2f=[chan2x+chan2y+chan2z]/3; %filtered data

chan3x=[x;chan3(1:(max(size(chan3))-1))]; %f(n-1)

chan3y=[chan3(1:max(size(chan3)))]; %f(n)

chan3z=[chan3(2:max(size(chan3)));z]; %f(n+1)

chan3f=[chan3x+chan3y+chan3z]/3; %filtered data

%*********************************************************************

x=0; y=0; z=0; D=time(2)-time(1);

velw=[x;y;chan3f(1:(max(size(chan3f))-2))];

velx=-8*[y;chan1f(1:(max(size(chan1f))-1))];

vely=8*[chan1f(2:max(size(chan1f)));x];

velz=-[chan1f(3:max(size(chan1f)));x;z];

velf=1/(12*D)*[velw+velx+vely+velz];

vel2w=[x;y;chan2f(1:(max(size(chan2f))-2))];

vel2x=-8*[y;chan2f(1:(max(size(chan2f))-1))];

vel2y=8*[chan2f(2:max(size(chan2f)));x];

vel2z=-[chan2f(3:max(size(chan2f)));x;z];

vel2f=1/(12*D)*[vel2w+vel2x+vel2y+vel2z];

vel3w=[x;y;chan1f(1:(max(size(chan1f))-2))];

vel3x=-8*[y;chan3f(1:(max(size(chan3f))-1))];

vel3y=8*[chan3f(2:max(size(chan3f)));x];

vel3z=-[chan3f(3:max(size(chan3f)));x;z];

vel3f=1/(12*D)*[vel3w+vel3x+vel3y+vel3z];

%*********************************************************************

figure;

plot(time,chan1f,'r');

hold on

plot(time,chan2f,'g');

hold on

plot(time,chan3f,'b');

grid on;

zoom on;

title('Graph of Joint Angle Against Time');

ylabel('Joint Angle (Degrees)');

xlabel('Time (Seconds)');

legend('chan1f','chan2f','chan3f');

figure;

subplot(2,1,1);

plot(time,chan1f,'r');

grid on;

zoom on;

ylabel('Joint Angle (Degrees)');

title('Graph of Channel 1');

subplot(2,1,2);

plot(time,velf,'b');

ylabel('Velocity');

grid on;

zoom on;

xlabel('Time (Seconds)');

figure;

subplot(2,1,1);

plot(time,chan2f,'b');

grid on;

ylabel('Joint Angle (Degrees)');

title('Graph of Channel 2');

subplot(2,1,2);

plot(time,vel2f,'g');

ylabel('Velocity');

grid on;

xlabel('Time (Seconds)');

figure;

subplot(2,1,1);

plot(time,chan3f,'g');

grid on;

ylabel('Joint Angle (Degrees)');

title('Graph of Channel 3');

subplot(2,1,2);

plot(time,vel3f,'b');

ylabel('Velocity');

grid on;

xlabel('Time (Seconds)');

%The Frequency Program (Number of Occurrences)

chan1a=round(chan1f);

countp=zeros(1,120);

countn=zeros(1,90);

countz=0;

for i = 1:max(size(chan1a))

if chan1a(i) < 0

countn(abs(chan1a(i)))=countn(abs(chan1a(i)))+1;

end

if chan1a(i) > 0

countp(chan1a(i))=countp(chan1a(i))+1;

end

if chan1a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(90:-1:1);

h=[cn countz countp];

hx=-90:1:120;

figure;

plot(hx,h);

grid on;

bar(hx,h);

zoom on;

title('Graph Showing the Frequency of Occurrences of the Joint   Angles for Channel 1');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

chan2a=round(chan2f);

countp=zeros(1,120);

countn=zeros(1,90);

countz=0;

for i = 1:max(size(chan2a))

if chan2a(i) < 0

countn(abs(chan2a(i)))=countn(abs(chan2a(i)))+1;

end

if chan2a(i) > 0

countp(chan2a(i))=countp(chan2a(i))+1;

end

if chan2a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(90:-1:1);

h=[cn countz countp];

hx=-90:1:120;

figure;

plot(hx,h);

bar(hx,h);

grid on;

zoom on;

title('Graph Showing the Frequency of Occurrences of the Joint Angles for Channel 2');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

chan3a=round(chan3f);

countp=zeros(1,120);

countn=zeros(1,90);

countz=0;

for i = 1:max(size(chan3a))

if chan3a(i) < 0

countn(abs(chan3a(i)))=countn(abs(chan3a(i)))+1;

end

if chan3a(i) > 0

countp(chan3a(i))=countp(chan3a(i))+1;

end

if chan3a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(90:-1:1);

h=[cn countz countp];

hx=-90:1:120;

figure;

plot(hx,h);

bar(hx,h);

grid on;

title('Graph Showing the Frequency of Occurrences of the Joint Angles for Channel 3');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

elseif num_of_chan == 2;

Gonio_title = input('Enter Goniometer Name for Channel 1:  ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle, c = %intercept (constant)= angle1, m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111;

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

% Program designed to remove 1 bit amplitude noise from the ELECTROGONIOMETRIC signal

chan1b = raw_data(1:2:length_of_data);

for i = 1:max(size(chan1b))-3

if (abs(chan1b(i) - chan1b(i+1)) == 1)

if (abs(chan1b(i+1) - chan1b(i+2)) == 1)

if chan1b(i)== chan1b(i+2)

chan1b(i+1) = chan1b(i);

end

else

if (abs(chan1b(i+1) - chan1b(i+2)) == 0)

if (abs(chan1b(i+2) - chan1b(i+3)) == 1)

if chan1b(i)== chan1b(i+3)

chan1b(i+1) = chan1b(i);

chan1b(i+2) = chan1b(i);

end

end

end

end

end

end

chan1 = (chan1b - angle1)/angle2; %Joint angles for chan1

Gonio_title = input('Enter Goniometer Name for Channel 2:  ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle, c = intercept (constant)= angle1, m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111; %not yet made

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

% Program designed to remove 1 bit amplitude noise from the ELECTROGONIOMETRIC signal

chan2b = raw_data(2:2:length_of_data);

for i = 1:max(size(chan2b))-3

if (abs(chan2b(i) - chan2b(i+1)) == 1)

if (abs(chan2b(i+1) - chan2b(i+2)) == 1)

if chan2b(i)== chan2b(i+2)

chan2b(i+1) = chan2b(i);

end

else

if (abs(chan2b(i+1) - chan2b(i+2)) == 0)

if (abs(chan2b(i+2) - chan2b(i+3)) == 1)

if chan2b(i)== chan2b(i+3)

chan2b(i+1) = chan2b(i);

chan2b(i+2) = chan2b(i);

end

end

end

end

end

end

% chan2 = (raw_data(2:2:length_of_data) - angle1)/angle2; %Joint   angles for chan2

chan2 = (chan2b - angle1)/angle2;

%*********************************************************************

x=0;

z=0;

chan1x=[x;chan1(1:(max(size(chan1))-1))]; %f(n-1)

chan1y=[chan1(1:max(size(chan1)))]; %f(n)                     chan1z=[chan1(2:max(size(chan1)));z]; %f(n+1)

chan1f=[chan1x+chan1y+chan1z]/3; %filtered data

chan2x=[x;chan2(1:(max(size(chan2))-1))]; %f(n-1)

chan2y=[chan2(1:max(size(chan2)))]; %f(n)

chan2z=[chan2(2:max(size(chan2)));z]; %f(n+1)

chan2f=[chan2x+chan2y+chan2z]/3; %filtered data

%*********************************************************************

x=0; y=0; z=0; D=time(2)-time(1);

velw=[x;y;chan1f(1:(max(size(chan1f))-2))];

velx=-8*[y;chan1f(1:(max(size(chan1f))-1))];

vely=8*[chan1f(2:max(size(chan1f)));x];

velz=-[chan1f(3:max(size(chan1f)));x;z];

velf=1/(12*D)*[velw+velx+vely+velz];

vel2w=[x;y;chan2f(1:(max(size(chan2f))-2))];

vel2x=-8*[y;chan2f(1:(max(size(chan2f))-1))];

vel2y=8*[chan2f(2:max(size(chan2f)));x];

vel2z=-[chan2f(3:max(size(chan2f)));x;z];

vel2f=1/(12*D)*[vel2w+vel2x+vel2y+vel2z];

%*********************************************************************

figure;

plot(time,chan1f,'r',time,chan2f,'b');

grid on;

zoom on;

title('Graph of Joint Angle Against Time');

ylabel('Joint Angle (Degrees)');

xlabel('Time (Seconds)');

legend('chan1f','chan2f');

figure;

subplot(2,1,1);

plot(time,chan1f,'r');

grid on;

ylabel('Joint Angle (Degrees)');

title('Graph of Channel 1');

subplot(2,1,2);

plot(time,velf,'b');

ylabel('Velocity');

grid on;

xlabel('Time (Seconds)');

figure;

subplot(2,1,1);

plot(time,chan2f,'g');

grid on;

ylabel('Joint Angle (Degrees)');

title('Graph of Channel 2');

subplot(2,1,2);

plot(time,vel2f,'b');

ylabel('Velocity');

grid on;

xlabel('Time (Seconds)');

%The Frequency Program (Number of Occurrences)

chan1a=round(chan1f);

countp=zeros(1,120);

countn=zeros(1,90);

countz=0;

for i = 1:max(size(chan1a))

if chan1a(i) < 0

countn(abs(chan1a(i)))=countn(abs(chan1a(i)))+1;

end

if chan1a(i) > 0

countp(chan1a(i))=countp(chan1a(i))+1;

end

if chan1a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(90:-1:1);

h=[cn countz countp];

hx=-90:1:120;

figure;

plot(hx,h);

bar(hx,h);

grid on;

zoom on;

title('Graph Showing the Frequency of Occurrences of the Joint Angles for Channel 1');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

chan2a=round(chan2f);

countp=zeros(1,120);

countn=zeros(1,90);

countz=0;

%for round(raw_data((length_of_data1+1)- angle1)/angle2):round(((max(size(raw_data)) - angle1)/angle2))

for i = 1:max(size(chan2a))

if chan2a(i) < 0

countn(abs(chan2a(i)))=countn(abs(chan2a(i)))+1;

end

if chan2a(i) > 0

countp(chan2a(i))=countp(chan2a(i))+1;

end

if chan2a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(90:-1:1);

h=[cn countz countp];

hx=-90:1:120;

figure;

plot(hx,h);

bar(hx,h);

zoom on;

grid on;

title('Graph Showing the Frequency of Occurrences of the Joint Angles for Channel 2');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

% Some statistics

mean1 = mean(chan1f);

fprintf('The mean for channel 1 is %g\n',mean1);

mean2 = mean(chan2f);

fprintf('The mean for channel 2 is %g\n',mean2);

min1 = min(chan1f);

fprintf('The minimum value for channel 1 is %g\n',min1);

min2 = min(chan2f);

fprintf('The minimum value for channel 2 is %g\n',min2);

max1 = max(chan1f);

fprintf('The maximum value for channel 1 is %g\n',max1);

max2 = max(chan2f);

fprintf('The minimum value for channel 2 is %g\n',max2);

range1 = max1 - min1;

fprintf('The range for channel 1 is %g\n',range1);

range2 = max2 - min2;

fprintf('The range for channel 2 is %g\n',range2);

else

Gonio_title = input('Enter Goniometer Name for Channel 1:  ','s');

error=0;

switch Gonio_title

%from calibration charts y=mx+c where y=length_of_data, x = angle, c = intercept (constant)= angle1 m = angle2

case{'s1','S1'}

angle1 = 129.46;

angle2 = 1.111;

case{'s2','S2'}

angle1 = 128.46;

angle2 = 1.111;

case{'s3','S3'}

angle1 = 127.92;

angle2 = 1.2957;

case{'s4','S4'}

angle1 = 126.89;

angle2 = 1.1054;

case{'s5','S5'}

angle1 = 130.16;

angle2 = 1.243;

case{'s6','S6'}

angle1 = 128.19;

angle2 = 1.1341;

case{'l1','L1'}

angle1 = 128.89;

angle2 = 0.857;

case{'l2','L2'}

angle1 = 130.38;

angle2 = 0.7558;

case{'l3','L3'}

angle1 = 127.84;

angle2 = 0.8165;

case{'l4','L4'}

angle1 = 127.49;

angle2 = 0.8776;

case{'l5','L5'}

angle1 = 127.84;

angle2 = 0.6332;

otherwise

error=1

end

chan1 = (raw_data(1:length_of_data) - angle1)/angle2; %Joint angles for chan1

%*********************************************************************

%filtered data using the moving average filter: [f(n-1) + f(n) + %f(n+1)]/3

x=0;

z=0;

chan1x=[x;chan1(1:(max(size(chan1))-1))]; %f(n-1)

chan1y=[chan1(1:max(size(chan1)))]; %f(n)       chan1z=[chan1(2:max(size(chan1)));z]; %f(n+1)

chan1f=[chan1x+chan1y+chan1z]/3; %filtered data

%************************************************************************************************

x=0; y=0; z=0; D=time(2)-time(1);

velw=[x;y;chan1f(1:(max(size(chan1f))-2))];

velx=-8*[y;chan1f(1:(max(size(chan1f))-1))];

vely=8*[chan1f(2:max(size(chan1f)));x];

velz=-[chan1f(3:max(size(chan1f)));x;z];

velf=1/(12*D)*[velw+velx+vely+velz];

%*******************************************************************

figure;

plot(time,chan1f,'r');

grid on;

title('Graph of Joint Angle Against Time');

ylabel('Joint Angle (Degrees)');

xlabel('Time (Seconds)');

hold on;

plot(time,velf,'b');

hold on;

legend('chan1f','vel','accel');

figure;

subplot(2,1,1);

plot(time,chan1f,'r');

grid on;

title('Graph of Joint Angle Against Time');

ylabel('Joint Angle (Degrees)');

subplot(2,1,2);

zoom on;

plot(time,velf,'b');

grid on;

ylabel('Velocity');

xlabel('Time(Seconds)');

grid on;

zoom on;

%The Frequency Program (Number of Occurrences)

chan1a=round(chan1f);

countp=zeros(1,90);

countn=zeros(1,90);

countz=0;

for i = 1:max(size(chan1a))

if chan1a(i) < 0

countn(abs(chan1a(i)))=countn(abs(chan1a(i)))+1;

end

if chan1a(i) > 0

countp(chan1a(i))=countp(chan1a(i))+1;

end

if chan1a(i) ==0

countz=countz+1;

end

end

%Program for plotting histogram

cn=countn(90:-1:1);

h=[cn countz countp];

hx=-90:1:90;

figure;

plot(hx,h);

figure;

bar(hx,h);

grid on;

zoom on;

title('Graph Showing the Frequency of Occurrences of the Joint Angles for Channel 1');

xlabel('Joint Angle (Degrees)');

ylabel('Number of Occurrences');

end

fprintf('To Analyse Specific Time Ranges from the Data use the following command,\n')

fprintf('axis([initial time range, final time range,J.angle1,J.angle2])\n')

fprintf('Peak values can be achieved by using the ginput function')

fprintf('\n\n\n\n')

%Program to indicate accurate angular measurements required

figure(1)

msg1=sprintf('Type in the command: [time,chan1f] = ginput(N), where N is the number of points requested for,');

msg1=[msg1,sprintf(' whilst time(s) and chan1f(Joint Angle) are the x and y coordinates of the recorded points. ')];

msg1=[msg1,sprintf('Using the mouse place the cursor at the points of interest on the graph ')];

msg1=[msg1,sprintf('and observe the numerical values, then press ')];

msg1=[msg1,sprintf('the LEFT mouse button and then the ENTER key.')];

h=msgbox(msg1);

waitfor(h)