Appendix 2B2

Tendon Travel Calculation

% A Predictive Model of Extrinsic Finger Flexor Tendon Wrist Joint

% Displacement For Wrists of Different sizes

% Tendon Displacement (mm) = 0.0263W1 + 0.0050W1W2 + 0.106W1W3 –

% 0.00096W1W2W4

% W1 - joint angle in degrees from neutral wrist position (W1>0, wrist

% flexion:

% W1<0 wrist extension)

% W2 - joint thickness (mm)

% W3 - 0(extension), 1(flexion)

% W4 - 0(superfacialis), 1(profundus)

% Method applied during tendon displacement calculation

% The tendon travel is calculated based on the maximum value from the 25

% samples/second signal. That is the max value in one second determines

% the furthest distance the tendon has travelled in that second.

ch3 = chan3f(2251:9750); % channel 3 array

ch3z = zeros(25,length(ch3)/25); % command sets a matrix of 25 rows

% (since frequency is 25Hz) and length(chan3f) columns in which data

% would be counted. Note all subjects must have the same experiment

% duration for command to be valid.

ch3z(:) = ch3;

%Flexion

ch3f = max(ch3z);

sumF = sum(abs(ch3f));

%Extension

ch3e = min(ch3z);

sumE = sum(abs(ch3e));

%Tendon travel calculations for wrist extension

%estimated from the max and min wrist joint angles from the 25 %samples/sec

WE = sumE; % total joint extension angle

WF = sumF; % total joint flexion angle

WEF = abs(sumE) + abs(sumF); % overall joint angle

W2 = input('Enter joint thickness (mm): ');

WE3 = 0;

WF3 = 1;

W4 = 0; %superfacialis

W4a = 1; %profundus

%Overall tendon displacement

TDEF = (0.0263*WEF)+(0.0050*WEF*W2)+(0.106*WEF*WE3)-(0.00096*WEF*W2*W4);

TDEFa =(0.0263*WEF)+(0.0050*WEF*W2)+(0.106*WEF*WE3)-(0.00096*WEF*W2*W4a);

fprintf('The tendon displacement(mm) (joint extension - superfacialis) is %g\n.',TDEF);

fprintf('The tendon displacement(mm) (joint extension - profundus) is %g\n\n.',TDEFa);

%Extension / Flexion

TDE = (0.0263*WE)+(0.0050*WE*W2)+(0.106*WE*WE3)-(0.00096*WE*W2*W4); % ext

TDEa = (0.0263*WE)+(0.0050*WE*W2)+(0.106*WE*WE3)-(0.00096*WE*W2*W4a);

TDF = (0.0263*WF)+(0.0050*WF*W2)+(0.106*WF*WF3)-(0.00096*WF*W2*W4); %flex

TDFa = (0.0263*WF)+(0.0050*WF*W2)+(0.106*WF*WF3)-(0.00096*WF*W2*W4a);

fprintf('The tendon displacement(mm) (joint extension - superfacialis) is %g\n.',TDE);

fprintf('The tendon displacement(mm) (joint extension - profundus) is %g\n.',TDEa);

fprintf('The tendon displacement(mm) (joint flexion - superfacialis) is %g\n.',TDF);

fprintf('The tendon displacement(mm) (joint flexion - profundus) is %g\n\n.',TDFa);