1.1. General

Modern DNA detection makes use of a number of techniques that are based on addition of the special labels to the DNA and their subsequent analysis. However, there is a constant search for new labelling and detection methods that will ease the chemistry involved and lower the detection limits. Surface enhanced resonance Raman scattering (SERRS) became a powerful technique throughout the last 15 years and it is now used in a wide number of analytical applications including the detection of DNA. Data presented in this thesis are the result of the work done to show the advantages of DNA detection by SERRS. The experiments performed indicate that there is potential for SERRS to be used not only in single and dual labelled DNA detection, but also in the DNA microarray field. Chapter 1 gives a general overview of the most significant work done by a number of researchers in the area of DNA labelling and detection, including a short look at the microarray field. Dual labelled oligonucleotides are also discussed because of their growing significance and potential for genotyping and the identification of genetic diseases. A short overview of polymerase chain reaction (PCR) technique is given as an example of the practical use of labelled oligonucleotides. Finally, the last section describes, in brief, the theory of SERRS as well as a range of applications.

1. 2. DNA Synthesis and Labelling

1.2.1. Synthesis of DNA

DNA is a key molecule for the transfer of information in living organisms.  Since the DNA model was described in 1953 (Fig. 1.1.)1, the chemistry and biology behind nucleic acids has been of interest to many research groups. With the completion of the human genome sequence in 2003, the first chapter of the DNA book was closed.2a,b However, a huge number of questions are still to be answered. Although, 50 years after the discovery of the double helix, we understand the role of DNA better, the post genomic era focuses further on the link between the DNA structure, our genes and the proteins they encode.

The genetic information is encoded in the precise sequence of bases along each strand of DNA and when replication takes place, the complementary strand acts as a template. The flow of information in the cells, controlled by DNA, consists of two basic steps; transcription and translation. In the transcription step, DNA serves as a template for RNA synthesis that in turn acts as a template for the protein synthesis in the subsequent process of translation.

When the importance of DNA was realised, scientists tried to synthesise DNA to investigate the link between its structure and the function of genes and the proteins that it encodes.  Nowadays, synthetic oligonucleotides are one of the most powerful tools in biotechnology. They are used as probes in measuring gene expression in DNA arrays3, as primers in polymerase chain reaction4, antisense agents in therapeutic applications5 and as molecular probes6 for detecting unknown DNA and genetic mutations.

Figure 1.1. The basic structure of DNA and formation of the double helix by Watson

Crick base pairing.

The successful solid phase synthesis of DNA used today is a product of the extensive research over the past 50 years and is a combination of the effective solid phase chemistry and phosphoramidite methodology first time used by Caruthers and Beaucage.7 In their paper published in 1981 they describe the reaction of two modified nucleosides giving the product with a 3’-5’ linkage in high yield.

DMTr – dimethoxytrityl (protecting group)

Figure 1.2. Formation of a 3’-5’ internucleoside linkage by Caruthers and Beaucage.7

As a starting material they used protected thymidine phosphoramidite and reported that this type of nucleoside can be successfully used in the solid phase synthesis of dinucleoside phosphates. Significant credit for the development of phosphoramidite methodology also goes to Letsinger and Lunsfort who were the first to report that P (III) was considerably more reactive than the corresponding P (V) acylating agents8 therefore providing the scientific base for the Caruthers and Beaucage (Fig. 1.2.) reaction and Koester et al.9 for their work done with 2-cyanoethyl phosphoramidites which are used in solid phase oligonucleotide synthesis.

# 1.2.2. Automated solid phase synthesis of DNA

To ensure the formation of a specific 3’-5’ linkage between two deoxynucloside units, all reaction centers not involved in the linkage must be protected.10 Protected deoxynucleosides are then added to a CPG (controlled pore glass) support (Fig 1.3):

Figure 1.3. The addition of protected nucleosides to CPG.

When added to CPG, the 5’- hydroxyl is deprotected by treatment with CHCl2COOH to remove the DMTr group. The deprotected deoxynucleoside is then coupled with the protected deoxynucleoside containing a phosphoramidite group at the 3’ alcohol. The phosphite product (PIII), which is very reactive, is oxidised to the phosphate triester by treatment with iodine and water (Fig 1.4).

Figure 1.4.  Principle of solid phase synthesis. 1) Coupling of the CPG

deoxynucleoside with deoxynucleoside phosphoramidite, 2) oxidation of

the phosphate product to the phosphate triester.

The desired sequence is built by adding the bases step by step using the same procedure in each cycle.  The final step involves the cleavage of all protecting groups and the ester bond, holding the DNA onto the CPG, by treatment with ammonia.

DNA does not have intrinsic properties that allow direct, highly sensitive detection by a range of analytical methods. Therefore, it is necessary to make DNA detectable using non-covalent or covalent labelling methods. Non-covalent labelling includes stains (ethidium bromide) or intercalator dyes (SybrGreen) that enable detection and quantification of DNA in solution11, immobilised in gels12 or blotted on membranes13. However, they do not provide any specific information about the sequence nor discrimination between different sequences present in the same solution. To enable the detection of specific DNA it is necessary to label it covalently by introduction of functional groups that ease the subsequent detection.

# 1.2.3. Labelling of DNA

The desired characteristics of the label are its sensitivity, ease of detection, stability and the overall low cost of the label and its detection technique. There is a range of methods available for DNA detection, but fluorescence14 and radioactive label detection15 are most widely used.   Although less sensitive, fluorescent dyes (Fig. 1.5) offer some advantages over radioactive labelling. One of the most important is the absence of radiation hazard and all related problems of waste handling and disposal. Moreover, several fluorescent dyes can be monitored in the same experiment and be detected in real time with high resolution.

Although fluorescence is a widely used method for DNA detection, especially in micorarrays, there are some disadvantages. Fluorescence is quantitative and highly sensitive and can be used to distinguish between labelled DNA strands. However, fluorescent dyes are difficult to synthesise and manufacture which makes them very expensive. Labelled strands also require the separation from the excess dye prior to analysis due to the overlapping fluorescence emission profiles. Moreover, some dyes are found to impair the biologically activity of DNA therefore effecting its analysis.16

Figure 1.5. Some of the commonly used fluorophores.

Apart from fluorescent dyes, fluorescent inorganic microparticle labels called quantum dots have been developed.17 Quantum dots (QD) have shell core construction, comprising of an outer shell of zinc sulphide surrounding the core containing a semiconductor material. Core composition and particle size determine the wavelength of emitted light. Highly luminescent CdSe QD’s are extensively used as non radioactive labels.18a,b They are small, stable under a range of conditions, produce intense signals and have a potential for multicolour detection.  However, the disadvantage still remains in the preparation of QD’s as well as their insolubility in polar solvents due to the organic coating on the surface, but future research might offer the solutions to these problems.

Recently, Fang et al. described a new method of DNA detection.19 They used anodic stripping voltammetry (ASV) to follow the hybridisation of a DNA probe labelled with the silver nanoparticle to a target DNA immobilized on a glassy carbon electrode (GCE). When a double strand is formed, silver nanoparticles linked to the double strands formed on the GCE surface are dissolved and the quantity of Ag(I) ions measured. The target DNA was detected in pmol/dm3 concentration and the electrochemical device designed is simple, compact and inexpensive. However, the lengthy preparation of immobilised GCE and labelled DNA still does not make it commercially viable. Other electrochemical DNA biosensors were also reported, all relying on the conversion of the hybridisation event into useful electrical signals.

All mentioned techniques require the attachment of a label to the oligo and the labels of interest are attached directly to the DNA probe during or immediately after automated solid phase synthesis.

The special labels that can be incorporated into DNA in vivo were also developed and they involve nucleotide modifications.20 One of the most widely used in vivo labelling procedures involves the modification of deoxyuridine. In the study by Cook at al. dUTP labelled with fluorescent Cy5 dye was used as a thymidine triphosphate analogue for in vivo labelling of DNA to enable direct imaging of DNA in living cells.21 Although the results of in vivo labelling are encouraging, the chemical synthesis of nucleoside triphosphates is complex and the determination of the right conditions for enzymatic reaction is a lengthy process. Therefore, preferred labelling methods used in laboratories around the world involve the incorporation of the labels by chemical means. The most commonly used methods are described below.

1.2.3 a) Labelling using amino link

Labelling of oligonucleotides through an amino link is a widely used post-synthetic approach. An alkyl amino phosphoramidite (Fig. 1.6) is routinely used in automated synthesis to introduce the primary amino group at the 5’ end.22 If label has to be attached at some other position of the DNA, internally or at the 3’, nucleosides with an amino containing arm23 or amino controlled pore glass24 (CPG) can be also used respectively.

Figure 1.6. Commercially available MMT-5’-C6 amino linker.

The most common way of introducing a label to an amino linked oligonucleotide was by using the appropriate isothiocyanates (Fig. 1.7.).25

Figure 1.7.  Labelling of amino linked oligonucleotide with an isothiocyanate

label.

However, the problem with many isothiocyantes, especially fluorophore isothiocyanates is their instability. Banks et al. investigated the stability of the fluorescein isothiocyanate – lysine conjugate and showed that there is significant hydrolysis of the thiourea bond, especcially if the tempreature is increased above 37ºC. 26

Another method, more favored, is use of an active ester, mainly a succinimidyl ester, which can be added to an amino linked oligonucleotide probe either in solution or on the solid phase (Fig. 1.8.). 27

Figure 1.8. Labeling of amino linked oligonucleotide with a succinimidyl label.

Although this method is popular, the disadvantage is the hydrolysis of the succinimidyl ester moiety that can happen in the coupling reaction where aqueous conditions are required. Moreover, the succinimidyl ester of some dyes like rhodamine tends to decompose upon prolonged storage. To avoid this happening with rhodamine dye, which is particularly useful in DNA labelling, Vinyak developed a milder in situ procedure for the activation of the dye.28 This involved activation of rhodamine carboxylic acid with HBTU and DIPEA and coupling to an amino linked oligonucleotide to achieve more than 85% coupling efficiency.

.

# 1.2.3.b) Labelling using a thiol linker

# A thiol linker can be introduced to the different positions of an oligonucleotide allowing the maleimide derivative of different labels to undergo Michael addition (Figure 1.9).

Figure 1.9. Michael addition of thiolated DNA to a label maleimide.

A range of fluorophore maleimides is commercially available and used in fluorescence detection. Thiol linked DNA is also widely used for surface attachment of DNA to the maleimide modified glass surfaces for production of DNA microarrays.29a,b Moreover, a new group of nanobiosensors was developed recently by labelling thiolated oligonucleotides with gold nanoparticles. Mirkin et al. developed heterogeneous colorimetric oligonucleotide detection system based on gold particles modified with alkylthiol oligonucleotides.30a,b In this system, dispersed oligonucleotide-modified gold nanoparticles were assembled into aggregated networks via hybridisation with the complementary target oligonucleotide. This process caused a change in colour thus enabling the detection of the hybridisation event.  As mentioned earlier, one more group of labels, quantum dots (QD), has a growing significance. QD’s are added to the oligonucleotide via a thiol linker. Mirkin et al. also succeeded in labelling thiol linked DNA probe with CdSe QD and determining the change in fluorescence upon the addition of the complementary strand.18b Thus, a thiol linker is used in DNA labelling with range of different labels (nanoparticles, fluorophores, QD’s, fluorescent proteins31) and in their attachment to different surfaces (CPG, silica nanoparticles, paramagnetic beads). However, the main problem is still the rapid oxidation of the thiol to the disulphide.

# 1.2.3 c) Direct coupling using label phosphoramidites or label

# CPG’s

1) The 5’ end labelling

A phosphoramidite of the desired label (Fig. 1.10) can be sometimes synthesized and used in solid phase synthesis of oligonucleotides.

Figure 1.10.  Phosphoramidite of the fluorescent labels.

Fluorescein phosphoamidite is a widely used fluorophore for 5’ labelling. Therefore it is not surprising that there is a number of papers investigating different routes to fluorescein phosphoramidite synthesis. Nelson et al. reported the preparation of a fluorescein thioisocyanate based, 2-aminobutyl-1,3-propendiol linked prhosphoramidite.32 Thiesen et al. were the first to synthesis FAM (5-carboxy fluorescein) phosphoramidite and used it in the solid phase synthesis of oligonucleotides.33  They reacted NHS ester of 5-carboxyfluorescein with amine derivative prepared from solketal and resulting amide was then converted to the phosphoramidite by phosphitylation. The coupling of this phosphoramidite to the 5’-hydroxyl of the support bound oligonucleotides proceeded in 70% yield. 5- and 6- Carboxyfluorescein phosphoramidites are today commercially available from range of companies.

Figure 1.11. FAM phosphoramidite as prepared by Theisen et al.33

Recently Adamczyk and his research group additionally simplified fluorescein phosphoamidite synthesis. 34 They used the 5-hydroxymethyl fluorescein, which is an intermediate in the synthesis of 5–aminofluorescein, to make a monofunctional phosphoramidite (Fig. 1.12) for the 5’ labelling in one step.

Figure 1.12. FAM phosphoramidite prepared by Adamczyk et al.34

A short linker between the fluorescein nucleus and the reactive phosphoramidite group did not have any effect on the oligonucleotide synthesis. Although they report high yields of phosphitylation and subsequent oligonucleotide synthesis, the preparation of the starting material takes 3 steps, of which the reduction of activated carboxyfluorescein to hydroxylfluorescein is the most important but least successful.

Rhodamine phosphoramidites were synthesised a few years after their fluorescein analogues due to the stability problems and complex chemistry exhibited by the rhodamine dyes. Lyttle et al. investigated the differences in rhodamine dye forms resulted from the disposition of the 3-carboxylate and succeeded in the synthesis of tetramethylrhodamine (TAMRA) phosphoramidite (Fig 1.13.). 35

Figure 1.13. TAMRA phosphoramidite as synthesised by Lyttle et al.35

The coupling efficiencies in the DNA synthesiser were estimated at 90%, which is reasonably good taking into account the sensitivity of rhodamine dyes to coupling conditions.

The phosphoramidites of a number of  other dyes are commercially available although very expensive. The synthesis of fluorescent phosphoamidites is difficult, usually requiring 4 or more steps, the yields are low and extensive purification is required. Moreover, phosphoramidites of some important fluorophores are difficult to prepare or not sufficiently stable.

2) Internal labelling

Internal labelling is even more complex than 5’ labelling. It involves derivatisation of a nucleoside phosphoramidite with the label itself or the functional groups which can undergo subsequent reaction with the label. Hagmar et al.23 synthesised an aminouridine phosphoramidite possessing a short alkylamino side arm which was used to react with fluorescein isothiocyanate. The yields for 1 mol synthesis of oligonucleotides containing the modified uridine base were about 50%. The other, improved approaches include the use of nucleotide phosphoramidite (dT is commonly used) derivatised with labels (Fig 1.14), which are commercially available (Glen Research)36 but at considerable cost.

Figure 1.14.  Fluorescein dT for internal labelling available from Glen Research. 36

3) The 3’ labelling – use of CPG supports

When 3’ labelling of the oligonucleotide is required, different types of CPG supports are used. Lyttle et al. prepared CPG containing 3’ terminal phosphodiesters modified with linkers bearing either an amine, thiol or hydroxy group (Fig.1.15). 24

These CPG supports can be used in the solid phase synthesis and the desired label added to the synthesised oligonucleotides after cleavage from the support.

Figure 1.15. CPG supports for oligonucleotide synthesis prepared by Lyttle.24

Mullah and Andrus derivatised a CPG support with TAMRA (Fig. 1.16) and used it in DNA synthesis. 37

Figure 1.16. TAMRA CPG.

They also investigated the conditions for cleavage of the rhodamine oligonucleotide from the solid support because of the rhodamine sensitivity towards the standard cleavage procedure using ammonia.

A range of CPG supports with different fluorophores and other labels like biotin are commercially available. However, a problem encountered with CPG is the sensitivity of labels to the cleavage conditions limiting the number of available CPG supports to the most stable fluorophores (FAM, TAMRA).

Although the use of direct coupling of labels through label phosphoramidites or CPG supports has the advantage of being a fast way of making labelled oligos, the reagents are expensive and often very sensitive to both the standard conditions of coupling in DNA synthesis and cleaveage from the solid support leading to lower yields. Therefore their use is limited and there is a constant search for new labelling methods.

1.2.3. d) Other labelling methods

There are many other methods for fluorescent labelling of DNA that utilise different chemical methodologies to achieve the goal. Dey and Sheppard synthesised nucleoside phosphoramidite with a ketone arm on the C-5 of uridine and then used it in solid phase synthesis to make a 17mer oligonucleotide.38 The ketone – DNA was then selectively modified with different aminooxy derivatives (glucose and biotin) (Fig. 1.17.) under physiological conditions.

Figure 1.17. Labelling via ketone DNA.38

However, although the oligonucleotide was successfully labelled without any side products, the procedure is time consuming. It takes 24 h at 37C for the reaction to be completed. Moreover, the synthesis of the ketone phosphoramidite required several steps, involving low yielding transformations.

The research group of J. Ju used “click chemistry” to construct a fluorescent oligonucleotide.39 They used the coupling between azides and alkynes as the labelling method. However, they still used amino linked DNA to make an azide and commercially available fluorescein NHS ester to make alkynyl-6-carboxyfluorescein therefore not cutting the time nor effort significantly. Moreover, the “click reaction” was carried out at 80C, which is too harsh for some labels and it took 72 h to reach completion.

Brown et al. recently reported the use of Fmoc protected alkyl alcohol modified tymidine phosphoramidite.40 Such phosphoramidite was incorporated into an oligonucleotide during routine automated solid phase synthesis, Fmoc group removed and the hydroxyl group coupled with commercially available dye phosphoramidite (Fig. 1.18.).

Figure 1.18. Modified tymidine phosphoramidite and its use in oligonucleotide

synthesis as reported by Brown et al.40

This method was particularly useful for addition of Cy5 dye to oligonucleotides. Commercially available Cy5 monomer, when added internally to an oligonucleotide, causes destabilisation of the duplex. However, destabilisation was significantly reduced by the use of modified tymidine and subsequent addition of Cy5 phosphoramidite.

Although there are many methods available for DNA labelling (as shown previously) there is still a need for faster, more effective and less expensive chemistry. The DNA microarray technology is developing rapidly and there is a need for fast screening of a large number of oligos, which requires sensitive detection techniques and simple labelling chemistry. The use of microarrays has revolutionised biotechnology and the DNA research and technology is now being applied to proteomics and drug testing. Microarrays are often referred to as “lab on the chip” since immobilised molecules retain the ability to undergo various reactions. DNA arrays were first introduced about 7 years ago, but the technology has developed and there are now three basic types: DNA, protein and chemical microarrays. A short overview of the DNA microarrays development and the chemistry and technology used in their production is given in the next section.