1.4.  Polymerase chain reaction (PCR)

Quantitative polymerase chain reaction (Q PCR) involves both amplification and analysis of a specific fraction of DNA with no need for additional sample manipulation.50a,b The method uses a pair of synthetic oligonucleotides as forward and reverse primers, each hybridising to one strand of a double stranded DNA (ds DNA) target. The hybridised primer acts as a substrate for a Taq DNA polymerase enzyme (source of which is the thermophilic bacterium Thermus aquaticus and therefore the name Taq), which creates a complementary strand via sequential addition of deoxynucleotides (Fig. 1.20).

1. Denaturation of double stranded DNA

2. Primer annealing

3. Primer extension

Figure 1.20. The principle of PCR.

As shown in the Fig. 1.20., the PCR process can be summarised in three steps:

1) denaturation of ds DNA  at temperatures higher than 90C

2) primer annealing at 50 - 75C

and	3) primer extension at 72-78 C

Detection of the PCR amplified DNA was originally based on the use of non-specific detection methods. These included post –amplification staining with ethidium bromide or use of intercalator dyes which bind to the double stranded DNA. However, the main drawback of these methods is that both specific and non specific products generate signal leading often to the false reads and misinterpretation of the results.51

The PCR system was improved by introducing specially designed oligonucleotide probes enabling the detection in real time and in a closed tube format, completely eliminating post amplification manipulation of the sample.

In 1991., Holland et. al. described the use of fluorogenic oligonuclotide probes in homogenous PCR.52 In their experiment the amplified sequences (amplicon) were detected by monitoring the effect of Taq DNA polymerase’s 5’3’ endonuclease activity on specific oligoprobe/target DNA duplexes. Lee at al. used this concept and introduced TaqMan probes in 1993 (Fig. 1.21.).53 A TaqMan probe containing a reporter and quencher dye is designed to anneal between the forward and reverse PCR   priming sites. During PCR, the probe anneals to the specific sequence location with high specificity and is subsequently cleaved by the endonuclease activity of the Taq DNA polymerase. When the probe is cleaved, the reporter dye is displaced and separated from the quencher dye which results in the increase of the fluorescence signal.

The fluorescence data collected indicated the extent of amplification. Since this type of assay involved measuring fluorescence without opening PCR tubes, the risk of carry over contamination was reduced. Furthermore, this type of assay is not labour intensive and is easily automated. However, when single nucleotide polymorphisms (SNP) have to be detected, TaqMan probes can assay only one to three SNP at the time.54

Figure 1.21. The principle of TaqMan. First step is annealing of forward and reverse

primer sequences and TaqMan probe containing reporter and quencher

dye.  Second step is extension of DNA sequences using TaqDNA

polymerase. During extension the TaqMan probe is cleaved  by the

endonuclease activity of the Taq DNA polymerase. This leads to the

displacement of the reporter dye and its separation from a quenching

dye, which results in the increase of the fluorescence signal.

The additional improvement of Q PCR amplicon detection came with the development of haripin oligonucleotides, molecular beacons (described in more detail in the following section). Amplifluor hairpin pimers55 and Scorpion probes 56a,b were also designed based on the molecular beacon concept. All of these probes provided higher specificity in SNP detection and, more importantly, enabled mulitplexing. The most widely used and simplest in design of these probes are molecular beacons where the simple concept proved highly effective in the detection of genetic polymorphisms and identification of genetic diseases.