SPORTS TURF TESTING RIG

Summary of operation:

The mechanical test rig designed to dynamically test sports turf surfaces (natural and artificial turf) by imparting loads in three directions to a test foot placed on the test surface.  The loads will mimic the loading that occurs between a player and the ground during various sporting activities.

The basic concept of the rig’s operation consists of two weighted pendulums that will be released from elevated positions. The pendulums consist of mass-spring systems. One of the pendulums will impart a vertical force to a vertical shaft, attached to the test foot. The shaft is supported within a centre block containing a combined linear and rotary bearing system.  The other pendulum impacts a L-shaped lever that transfers the applied vertical force to a horizontal force via a cable attached to the vertical shaft by an eyebolt.  The centre block also contains two horizontal shafts contained within linear bearings to allow the horizontal motion of the test foot.  The cable attached to the vertical shaft can also be offset away from a central position to allow a torque to be generated by the horizontal force.  The test foot is allowed to rotate about a vertical axis in the combined linear/rotary bearings.

Each pendulum can have additional masses secured to them and are released from the elevated positions by electromagnets, triggered by an impulse from a LabView programme.

The area of the test foot contact area is that of a UK size 10 shoe. The test foot can have two types of sole attached, depending on the type of surface to be tested. One sole consists of 4 standard rubber studs (cleats) screwed into an aluminium plate. The other test foot consists of the pimpled rubber material from the sole of an Umbro® artificial surface football shoe

A pylon transducer located just above the test foot measures the forces applied by the test foot to the surface.  Strain gauges on the pylon transducer will measure vertical and horizontal forces and axial torques.  The vertical and horizontal acceleration of the test foot will be measured by three accelerometers located within the housing of the pylon transducer. The analogue signals from the transducer will be directed through battery powered strain gauge amplifiers and collected by an A/D card in a laptop, running LabView.

The testing rig is designed to be portable, so that different areas of the test pitch can be assessed.  Four, large wheels with pneumatic tyres are situated on the four corners of the rig, with a steering system at one end of the rig.  During the testing situation, the rig is jacked up onto four legs, which are secured in place to provide a rigid base of support.

Turf Characterisation Test Battery

Calculated Applied Loads

| Direction | Pendulum Mass (kg) | Load |
| --- | --- | --- |
| Vertical | ~19.4 | 3500N |
| Vertical | ~9.1 | 2400N |
| Horizontal | ~6 | 1600N |
| Horizontal | ~3.4 | 1200N |
| Torque | ~3.4 (horiz) | 39Nm |

# TEST 1: LINEAR TRACTION

Input variables:

- Static 19.4kg vertical pendulum

- Dynamic 6kg horizontal force

- Zero torque

TEST 2: ‘DYNAMIC’ LINEAR TRACTION (STOPPING)

Input variables:

- Dynamic 19.4kg vertical force

- Dynamic 6kg horizontal force

- Zero torque

# TEST 3: COMBINED LINEAR & ROTATIONAL TRACTION

Input variables:

- Static 19.4kg vertical force

- Dynamic 3.4kg horizontal force

- Torque

# TEST 4: VERTICAL IMPACT TEST

Input variables:

- 19.4kg vertical force

- Zero horizontal & torque force

TEST 5: HIGH DYNAMIC TASKS

Input variables:

- Dynamic 19.4kg vertical force

- Dynamic 3.4kg horizontal force

- Torque

TEST 6: LOW DYMANIC TASKS

Input variables:

- Dynamic 9.1kg vertical force

- Dynamic 3.4kg horizontal force

- Torque