Subject Information Sheet

Investigation Title

Testing the reliability of the newly developed user friendly flexible Electrogoniometry system (SUDALS) against the commercially available Biometrics data logging system during various activities of daily living

Purpose of the Study

The study aims to test reliability of the user friendly system of flexible Electrogoniometry system developed (SUDALS) against the commercially available Biometrics data logging system, on 10 volunteer subjects during various activities of daily living (walking, stair climbing, ramp climbing, deep squatting and in and out of a chair). Further, this investigation would give us an idea of how the developed system would be useful in  clinical applicability and multi centred clinical trials. In addition to this, it would also give us better idea of its suitability for assessing the functional abilities of patients who have undergone Total knee replacement.

Who Should Volunteer?

This study requires 10 Volunteers, both males and females in age group of 20 to 30. No incentives are offered to volunteers. The exclusion criteria would be:

- Any active illness.

- Previous history of Osteoarthritis or Total Knee Arthroplasty

- Diagnosed skin condition

- Known allergy to sticking plasters or tape

- Taking any kind of medication that may affect alertness or balance

Procedure

Two Flexible electrogoniometer will be attached to the lateral side of the right and left knee through attachment strips which in turn are fixed to the skin using hypo allergenic adhesive tape. Further, four force sensing resistors, two on each foot between the foot and the shoe interior will be used to determine foot contact with the floor. These transducers would be interfaced with the battery powered user friendly data logging system (SUDALS) placed in a pouch and hung on the back or front of the subject depending upon their convenience.Each subject has the above attachment placed on them. With these attachments, the subject is asked to perform the various activities of daily living in an unconstrained environment. During each of these activities, the information pertaining to the flexion - extension of the knee from flexible electrogoniometer and the event marking from force sensing resistors would be stored in the user friendly system, which would be then transmitted to a PC via Bluetooth transceiver. The entire range of activities mentioned above would be carried out in the biomechanics movement analysis laboratory in the bioengineering unit. Following this, with all other sensors left dislodged, only the SUDALS will be replaced by the commercially available Biometrics data logging system and the above procedure will be repeated to collect data pertaining to the flexion/extension of knee during the various above mentioned ADL’s.  The duration of the trial would approximately 2-3 hours and would be carried out either in the morning or in the afternoon according to the convenience at the movement analysis lab in the Bioengineering unit.

Possible Hazards:

- Tripping or Slipping: Subjects will be performing activities of daily living and therefore the risk is no higher than being in your own home.

- Allergic reaction to tapes: This is low risk as hypoallergenic tape is used.

- Electrocution: This is low risk as the transducers require only 5V D.C supply and they are all battery operated. Further, there is no mains connection during data transmission as Bluetooth is used for this application.

Procedures to minimise the risk caused by hazards:

- To reduce the risk of slipping or tripping on the stairs a contrast edge has been painted around each step to aid visual input. Handrails are present, if in case the subject loses balance.

- All the areas will be cleared of obstruction and the surfaces would be kept dry.

- All the objects will be marked so that their edges are clearly visible. Subjects with visual or balance defects will be excluded from testing to reduce risk of falls.

- To reduce the risk of reaction to adhesive tape, subject will be asked if they have any allergies to tape and will be excluded if this is the case. Skin checks will be performed through out the tests to ensure there is no irritation.

All information would be considered confidential and will be used anonymously for research purposes only. If you have any queries regarding the project, please feel free to contact me at the address given below:

Principal Investigator:

I.Vivek Padmanaabhan,

Postgraduate Researcher,

Department of Bioengineering,

University Of Strathclyde,

106, Rottenrow,

Wolfson Centre,

Glasgow – G4 0NW.

Email: vivek_vip2000@yahoo.co.in