Subject Information Sheet

Investigation Title

Evaluation of the user friendly nature of Strathclyde university data logging system (SUDALS) during various activities of daily living (ADL) such as walking, deep squatting, getting in and out of a chair based on                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                             the user feedback following the data collection process.

Purpose of the Study

The purpose of this study is to:

To evaluate the user-friendly nature of SUDALS during various above-mentioned ADL by giving training to five health professionals in the use of conventional electrogoniometry system and the new system, allow them to collect data with the systems from one healthy normal volunteer and elicit their views on the strengths and weakness of both systems. This would help us to improve the effectiveness, safety and usability of the device based on the user feedback.

Who Should Volunteer?

For the purpose of  intra-rater and inter-rater reliability, this study requires one healthy normal volunteer, either male or female in age group of 20 to 30. A national minimum wage for the calculated amount of time that was spent during this study would be paid to the subject.The exclusion criteria would be:

- Any active illness.

- Previous history of Osteoarthritis or Total Knee Arthroplasty

- Diagnosed skin condition

- Known allergy to sticking plasters or tape

- Taking any kind of medication that may affect alertness or balance

- Subjects with visual or balance defects will be excluded from testing to reduce risk of falls.

Procedure

A Flexible electrogoniometer is a device or an instrument used for measuring the range of movement of a joint. Here in this study, two Flexible electrogoniometer will be attached to the lateral side of the right and left knee through attachment strips which in turn are fixed to the skin using hypo allergenic adhesive tape. Further, four force sensing resistors, two on each foot between the foot and the shoe interior, will be used to determine the foot contact with the floor. These transducers will be interfaced with the battery powered data collecting systems placed in a pouch and hung on the back or front of the subject depending upon their convenience.Each subject has the above attachment placed on them. With these attachments, the subject has to perform the various activities of daily living in two different sessions. During session 1(morning) the information pertaining to the flexion-extension of the knee from flexible electrogoniometers would be collected using the commercial data collecting system by a research nurse or a physiotherapist. Similarly, in session2 (afternoon) the same user is expected to collect data from the same volunteer during these ADL using SUDALS. The entire range of activities mentioned above will be carried out in the biomechanics movement analysis laboratory 3 in the bioengineering unit. The duration of the experiment will be 6-7 hours and the trials would be carried out in the lab mentioned above with sufficient breaks as and when required by the volunteer. For the purpose of intra-rater and inter-rater reliability study , the systems will be tested on a single subject by all the users on 5 different working days. Hence, the  subject should be willing to come on all 5 different working days.  The days on which the experiment would be taking place will be intimated  in advance to the volunteer according to the convenience of both the parties.

Note: Shorts will be provided to the subject during the experiment. However, the subject is expected to bring his/her own comfortable shoes.

Possible Hazards:

- Trips or Falls: Subjects will be performing activities of daily living and therefore the risk is no higher than being in your own home.

- Allergic reaction to tapes: This is low risk as hypoallergenic tape is used.

- Electrocution: This is low risk as the transducers require only 5V D.C supply and they are all battery operated. Further, there is no mains connection during data transmission as Bluetooth is used for this application. However, since three 9V rechargeable batteries are used to provide a regulated 5V supply, it is advisable to remove these batteries and recharge them before re-using them

Procedures to minimise the risk caused by hazards:

- To reduce the risk of slipping or tripping on the stairs a contrast edge is painted around each step to aid visual input. Handrails are present, if in case the subject loses balance.

- All the areas will be cleared of obstruction and the surfaces would be kept dry.

- All the objects will be marked so that their edges are clearly visible. Subjects with visual or balance defects will be excluded from testing to reduce risk of falls.

- To reduce the risk of reaction to adhesive tape, subject will be asked if they have any allergies to tape and will be excluded if this is the case. Skin checks will be performed through out the tests to ensure there is no irritation.

All information would be considered confidential and will be used anonymously for research purposes only. If you have any queries regarding the project, please feel free to contact me at the address given below:

Principal Investigator:

I.Vivek Padmanaabhan,

Postgraduate Researcher,

Department of Bioengineering,

University Of Strathclyde,

106, Rottenrow,

Wolfson Centre,

Glasgow – G4 0NW.

Email: vivek.indra-mohan@strath.ac.uk