3.3Microcontroller Firmware code

#include <ADuC7026.h>

extern int write (int file, char * ptr, int len); // Functions used to output data //

extern int getchar (void);

extern int putchar(int);      // Write character to Serial Port

void menu();

void calibrate();

unsigned short * data_ptr;

void delay(int);

void delay1(int);

void ADCpoweron(int);

void transmit();

void IRQ_Handler(void) __irq;

void overflowindicator();

int onflag = 0;

int chn = 1;

int click1 = 0;

int click2 = 0;

int click3 = 0;

int i = 0 ;

int doneflag = 0;

int expcounter = 0x00;

int Transmitflag = 0;

int transmitcounter = 1;

int calib1 = 0x0000;

int calib2 = 0x0000;

int offset1 = 0x0000;

int offset2 = 0x0000;

int toffset1 = 0x0000;

int toffset2 = 0x0000;

int samplecount[30];

unsigned short * data_ptr;    // Conversion information stored here //

unsigned short * new_data_ptr;

unsigned short * temp_data_ptr;

int yesnoflag = 0;

int stopflag = 0;

int busyflag = 0;

int scrapflag = 0;

int recordflag = 0;

int calibflag = 0;

int transmitflag = 0;

int resetflag = 0;

int N = 0x1000;

int k;

int jchar;

int eventcount = 0;

int memorydata = 0;

unsigned short tempdata;

unsigned short tempdata1;

unsigned short tempdata2;

unsigned short tempdata3;

unsigned short tempdata4;

int keypresscount = 0x00;

void main (void)

{

int i= 0;

data_ptr = (unsigned short *)0x10000000;

ADCpoweron(20000);                      // power	on ADC

GP0CON = 0x00000000;

GP2CON = 0x00000000;

DAC0CON = 0x12;                         // AGND-AVDD range

DAC1CON = 0x12;                         // AGND-AVDD range

DAC2CON = 0x12;                         // AGND-AVDD range

DAC3CON = 0x12;                         // AGND-AVDD range

REFCON = 0x01;                          // internal 2.5V reference

T1LD = 0x8000;                          // Counter Value for 20ms = corresponds to 50 Hz. derived from 20000 for50ms.

T1CON = 0x04;                           // Timer register, initially disabled, Periodic, Binary and CLK/16

IRQEN = XIRQ0_BIT+GP_TIMER_BIT;         //IRQEN is an interrupt register and it provides the value of the current enable mask. When bit                                                              // is set to 1, the source request is enabled to create an IRQ                                                              //exception. When bit is set to 0, the source request is disabled or                                                                //masked, which does not create an IRQ exception.

//Setup  tx & rx pins on P1.0 and P1.1

GP1CON = 0x011;                          // Start	 setting up UART at 115200bps

COMCON0 = 0x080;                      // Setting DLAB

COMDIV0 = 0x044;                       // Setting DIV0 and DIV1 to DL calculated

COMDIV1 = 0x000;

COMCON0 = 0x007;                        // Clearing DLAB

GP0CON = 0x220;                         // ext mem mode for BHE & BLE

GP2CON = 0x00022220;

GP3CON = 0x22222222;

GP4CON = 0x22222222;

XMCFG = 0x1;                       // to enable the memory register

XM0CON = 0x3;                     // to enable and control a region in the register

XM0PAR = 0x70FF;  //to start	accessing that particular region in the reg.;

DAC0DAT = 0x08100000;

DAC1DAT = 0x08100000;

DAC2DAT = 0x08100000;

DAC3DAT = 0x00000000;

while(1)

{

if (busyflag != 1)

{

menu();

}

}

}

void overflowindicator()

{

int loop;

loop = 0;

while(loop <5)

{

loop ++;

//make leds glow

GP0DAT = 0x48480000;

GP2DAT = 0x68680000;

delay(10000000);

//make leds off

GP0DAT = 0x48000000;

GP2DAT = 0x68000000;

delay(10000000);

}

return;

}

void delay1 (int length)

{

while (length >=0)

{

length--;

if(length >=1999999 && length <= 2000001)	// Buzzer   Functionality for 10% of total delay time.

{

if (yesnoflag  != 1 )

{

DAC3DAT = 0x08100000;

}

}

}

DAC3DAT = 0x00000000;

}

void delay (int length)

{

while (length >=0)

length--;

}

void ADCpoweron(int time)

{

ADCCON = 0x620;   // power-on the ADC

while (time >=0)  // wait for ADC to be fully powered on

time--;

}

void calibrate()

{

DAC0DAT = 0x08100000;

DAC1DAT = 0x08100000;

DAC2DAT = 0x08100000;

delay(2000000);

ADCCP = 0x01;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x01;

calib1 = ((ADCDAT >> 16)& 0x00000FFF);

calib1 = (calib1*1.00583329)- 10.58217136;

DAC0DAT = ((calib1 << 16)& 0x0FFF0000);

DAC2DAT = 0x08100000;

ADCCP = 0x02;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x02;

calib2 = ((ADCDAT >> 16)& 0x00000FFF);

calib2 = (calib2*1.014774959)- 16.018;// obtained equation

DAC1DAT = ((calib2 << 16)& 0x0FFF0000);

DAC0DAT = ((calib1 << 16)& 0x0FFF0000);

DAC2DAT = 0x08100000;

busyflag = 0;

return;

}

void menu()

{

yesnoflag = 0;

GP2DAT = 0x80800000;	// Pin 0.3

calibflag = 1;

delay1(20000000);

if (yesnoflag != 0)

{

// zeroing

busyflag = 1;

yesnoflag = 0;

calibrate();

DAC3DAT = 0x00000000;

GP2DAT = 0x80000000;

return;

}

calibflag = 0;

DAC3DAT = 0x00000000;

GP2DAT = 0x80000000;

GP0DAT = 0x08080000; // Pin 0.3

delay1(20000000);

recordflag = 0;

if (yesnoflag != 0)

{

// call recording function

recordflag = 1;

busyflag = 1;

stopflag = 0;

expcounter = expcounter+1;

samplecount[expcounter] = 0x0000;

T1CON = 0xC4;

DAC3DAT = 0x00000000;

GP0DAT = 0x08000000;

return;

//while (recordflag != 0)

//{}

}

DAC3DAT = 0x00000000;

GP0DAT = 0x08000000;

if ( expcounter>0 )

{

GP0DAT = 0x40400000;// Pin 0.6

scrapflag = 1;

delay1(20000000);

if (yesnoflag != 0)

{

expcounter = expcounter-1;

data_ptr = (data_ptr)- samplecount[expcounter]*3; // call scrap recording function

GP0DAT = 0x40000000;

return;

}

scrapflag = 0;

GP0DAT = 0x40000000;

GP2DAT = 0x20200000;

transmitflag = 1;

delay1(20000000);

if (yesnoflag != 0)

{

busyflag = 1;

GP2DAT = 0x20000000;

// call transmit function

transmit();

busyflag = 0;

data_ptr = (unsigned short *)0x10000000;

return;

}

transmitflag = 0;

GP2DAT = 0x20000000;

GP2DAT = 0x40400000;

resetflag = 1;

delay1(20000000);

if (yesnoflag != 0)

{

while(GP0DAT & 0x00010){}

RSTSTA = 0x4; // call reset function

GP2DAT = 0x40000000;

return;

}

resetflag = 0;

GP2DAT = 0x40000000;

}

return;

}

void IRQ_Handler() __irq    // example with two IRQ

{

if ((IRQSTA & XIRQ0_BIT) != 0) 	// external IRQ0

{

yesnoflag = 1;

if (recordflag != 0)

{

//DAC3DAT = 0x00000000;

stopflag = 1;

yesnoflag = 0;

delay(2000000);

}

while(GP0DAT & 0x00010){}

}

if ((IRQSTA & GP_TIMER_BIT) != 0) // Timer1 IRQ?

{

T1CLRI = 0;

ADCCP = 0x01;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x01;

tempdata = ((ADCDAT >> 16)& 0x00000FFF);

*data_ptr = tempdata;

data_ptr++;

ADCCP = 0x02;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x02;

tempdata = ((ADCDAT >> 16)& 0x00000FFF);

*data_ptr = tempdata;

data_ptr++;

ADCCP = 0x03;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x03;

tempdata1 = ((ADCDAT >> 16)& 0x00000FFF);

ADCCP = 0x07;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x07;

tempdata2 = ((ADCDAT >> 16)& 0x00000FFF);

ADCCP = 0x05;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x05;

tempdata3 = ((ADCDAT >> 16)& 0x00000FFF);

ADCCP = 0x06;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x06;

tempdata4 = ((ADCDAT >> 16)& 0x00000FFF);

// performing data compression

tempdata1 = ((tempdata1 & 0x0800)<<4);

tempdata2 = ((tempdata2 & 0x0800)<<0);

tempdata3 = ((tempdata3 & 0x0800)>>4);

tempdata4 = ((tempdata4 & 0x0800)>>8);

tempdata = ((tempdata1|tempdata2)|(tempdata3|tempdata4));

*data_ptr =  tempdata;

//bit wise 'or' operation of all the FSR signals

data_ptr++;

samplecount[expcounter] = samplecount[expcounter] + 1;

if (data_ptr >= 0x10008000)

{

T1CON = 0x04;

overflowindicator();

//remove last recording since it was not completed.

expcounter = expcounter-1;

data_ptr = (data_ptr)- samplecount[expcounter]*3;

transmit();

RSTSTA = 0x04;

}

if(stopflag!=0)

{

busyflag = 0;

stopflag = 0;

T1CON = 0x04;

recordflag = 0;

yesnoflag= 0;

return;

}

}

return ;

}

void transmit()

{

new_data_ptr = (unsigned short *)0x10000000;

while(!(0x020==(COMSTA0  & 0x020))){}

COMTX = expcounter;

for (i = 1;i <= expcounter;i++)

{

tempdata = samplecount[i];

while(!(0x020==(COMSTA0  & 0x020))){}

COMTX = 0x0D;

while(!(0x020==(COMSTA0  & 0x020))){}

COMTX = ((tempdata >> 8) & 0x00FF);

while(!(0x020==(COMSTA0  & 0x020))){}

COMTX = (tempdata &  0x00FF);

}

for (i = 0; i<32768;i++)//32768 - size of external memory

{

tempdata = *new_data_ptr;

while(!(0x020==(COMSTA0  & 0x020)))

{}

COMTX = 0x0D;

while(!(0x020==(COMSTA0  & 0x020)))

{}

COMTX = ((tempdata >> 8) &  0x00FF);

while(!(0x020==(COMSTA0  & 0x020)))

{}

COMTX = (tempdata &  0x00FF);

new_data_ptr++;

}

}