## 3.2 Explanation firmware logic

Zeroing Subroutine

In this subroutine, the DAC registers are initialised and the address of the DAC registers correspond to the zero factor as explained earlier in section 3.1.2. Prior to obtaining data from the flexible electrogoniometers, the ADC channels are initialised again by assigning suitable addresses. The two flexible electrogoniometers are connected to the positive ADC channels 1 and 2 and also prior to zeroing the sensors, it’s ensured that the ADC is not busy. This is implemented with the help of the ‘Infinite While Loop’. Once, the condition is satisfied, the data from the flexible electrogoniometers are obtained and sampled. The sampled data is stored in the ADCDAT register and it is assigned to a variable ‘calib1’. This sampled data obtained from the flexible electrogoniometer connected to ADC 1 is then multiplied with the calibration coefficients obtained (explained in 3.1.2). This in turn gives us the zeroing factor required, which is then assigned to the DAC0 channel. The format used for assigning the sampled data to the ADCDAT register is called as the rotating format. Basically, the sampled data from the ADC channel is rotated to the right by 16 times and then to extract the data without any change it’s logically AND’ed with 0x00000FFF.Since the data is rotated to right by 16 times, when assigning the data to the DAC register, the data corresponding to the zeroing factor has to be again rotated to the left by 16 times and should be AND’ed with 0x00000FFF to make the data in a readable format at the DAC end. During this whole process, the MCU is involved in performing a task; hence the busyflag status which is set to 1 is then made to 0 before transferring the control to the main program. The busyflag function and the delay function are called from the main function only when required. The Zeroing function being a part of the Void menu ( ), only when the busyflag status is equal to zero, the control is transferred to the required sub-functions within the main menu. Otherwise, the control is stuck within this infinite while loop itself.

The delay function makes use of a count down timer which creates the required delay in between the execution for a specified time length. This is generally useful for avoiding ‘firmware hang’ during the execution.

Recording Subroutine:

GP0DAT = 0x08080000; // Pin 0.3

delay1(20000000);

recordflag = 0;

if (yesnoflag != 0)

{

recordflag = 1;

busyflag = 1;

stopflag = 0;

expcounter = expcounter+1;

samplecount[expcounter]= 0x0000;

T1CON = 0xC4;

DAC3DAT = 0x00000000;

GP0DAT = 0x08000000;

return;

}

DAC3DAT = 0x00000000;

GP0DAT = 0x08000000;

Similar to the delay function explained above, the delay 1 function used here in this subroutine is used to provide the specific function time out indication by means of a ‘beep’ using a buzzer connected to the DAC4. This function is activated in such a way that, the buzzer goes high for a time period of 10% of total delay time, towards the end of the LED glowing period. This in turn prevents the user from clicking an unwanted function by mistake. This function is included in all the data logger function subroutines. Enabling the T1CON in this subroutine routes the control to the IRQ handler where the sampling of all the 6 channels takes place as shown below.

- IRQ Handler and Sampling routine

Void IRQ_Handler () __irq

{

If ((IRQSTA & XIRQ0_BIT)! = 0)

{

yesnoflag = 1;

if (recordflag != 0)

{

stopflag = 1;

yesnoflag = 0;

delay(2000000);

}

while(GP0DAT & 0x00010){}

}

if ((IRQSTA & GP_TIMER_BIT) != 0)

{

T1CLRI = 0;

ADCCP = 0x01;

ADCCON = 0x00A3;

While (!ADCSTA) {}

ADCCP=0x01;

tempdata = ((ADCDAT >> 16)& 0x00000FFF);

*data_ptr = tempdata;

data_ptr++;

k=k+1;

ADCCP = 0x02;

ADCCON = 0x00A3;

While (!ADCSTA) {}

ADCCP=0x02;

tempdata = ((ADCDAT >> 16)& 0x00000FFF);

*data_ptr = tempdata;

data_ptr++;

ADCCP = 0x03;

ADCCON = 0x00A3;

While (!ADCSTA) {}

ADCCP=0x03;

tempdata1 = ((ADCDAT >> 16)& 0x00000FFF);

ADCCP = 0x04;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x04;

tempdata2 = ((ADCDAT >> 16)& 0x00000FFF);

ADCCP = 0x05;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x05;

tempdata3 = ((ADCDAT >> 16)& 0x00000FFF);

ADCCP = 0x06;

ADCCON = 0x00A3;

while(!ADCSTA) {}

ADCCP=0x06;

tempdata4 = ((ADCDAT >> 16)& 0x00000FFF);

tempdata1 = ((tempdata1 & 0x0800)<<4);

tempdata2 = ((tempdata2 & 0x0800)<<0);

tempdata3 = ((tempdata3 & 0x0800)>>4);

tempdata4 = ((tempdata4 & 0x0800)>>8);

tempdata=((tempdata1|tempdata2)|(tempdata3|tempdata4));

*data_ptr =  tempdata;

data_ptr++;

samplecount[expcounter] = samplecount[expcounter] + 1;

Memory full Subroutine

if (data_ptr >= 0x10008000)

{

T1CON = 0x04;

overflowindicator();

expcounter = expcounter-1;

data_ptr = (data_ptr)- samplecount[expcounter]*3;

transmit();

RSTSTA = 0x04;

}

if(stopflag!=0)

{

busyflag = 0;

stopflag = 0;

T1CON = 0x04;

recordflag = 0;

yesnoflag= 0;

return;

}

}

return ;

}

Generally, the microcontroller should be able to perform any of the five functions, when it receives an external interrupt from the remote control circuit interfaced with the GPIO pin 0.4. This condition is always checked at the beginning of the IRQ handler routine and if this condition is satisfied, then the ‘yesnoflag’ is set high. This flag condition is always checked in the beginning of any subroutine associated with the functions of the data logger. Following this the IRQEN / Timer bit condition is checked. If this condition is satisfied, only then the sampling or data conversion process is initiated. This is very important condition as the above loop and the recording subroutine work simultaneously to record and convert the data collected from the transducers interfaced with the SUDALS. As explained above, once the data is sampled and converted to a digital value, the resultant is stored in the ADCDAT register and then following the rotation and logical anding, the data from the ADC channel 1 is stored in the specific memory location using the ‘*data_ptr’ variable. The pointer position is then incremented to store the processed data from the next ADC channel. This is same for all the ADC channels used here in this application. However, for the FSR channels, a data compression technique is implemented to make economical use of the available external memory. The data from all the four FSR channels; ADC3, 4, 5 and 6 are compressed to a single channel data.

The 12 bit data corresponding to each channel is compressed to 4 bits or the size of the data corresponding to 4 channels is reduced to 2 bytes. This is implemented by compressing the main significant nibble (MSB) of the data obtained from the ADC. If the MSB is 0 then the FSR value is considered to be low else it’s considered to be high. To extract this information, the MSB is logically AND’ed with 0x0800 (A threshold value approximately equivalent to 1.25 V chosen between the output range of the FSR – (0 to 5V or 000 to FFF) to result in a value equivalent to 0 or F (i.e. low or high). Then this is rotated by 4bits or 8 bits to left or right so that the values corresponding to that specific channel are arranged accordingly. For example, value corresponding to channel 3 should come first in the left most corner of the 2 byte array and the value corresponding to channel 6 should come in the right most corner of this array and similarly, the other values corresponding to channels 4 and 5 have to take their suitable positions in the array. The resultant array corresponding to channel 1, 2 and 3 will be as shown below. These data are stored in the respective variables tempdata1, 2, 3 and 4. Finally, performing a bitwise OR between the data obtained from channel 3, 4, 5 and 6 results in a single value (low or high) which is stored in the variable ‘tempdata’.

| Channel 1 | Channel 2 | Channel 3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Goniometer 1 | Goniometer 2 | Ch3 | Ch4 | Ch5 | Ch6 |
|  |  | fsr1-0/F | fsr2- 0/F | fsr3- 0/F | fsr4- 0/F |

Figure 3.2 – Data array showing the position of data corresponding to each channel.

The ‘tempdata’ now consists of data from all the four FSR channels compressed to a single channel and this is stored in the suitable memory location. At certain times, due to prolonged recording of data from all the 6 channels, the external memory in which all the data are stored will become full and this in turn can result in the loss of data even before the completion of data collection. Hence, to avoid such loss of data, when the memory becomes full, the ‘memory full’ subroutine included as part of the IRQ handler routine is executed and the data recorded until the current recording is transmitted automatically to the PC and the system is reset. This is also accompanied by a visual indication by the data logger to alert the user and stop the users from making further recordings until the system is completely reset.

As mentioned earlier, the external memory can store 32768 words of each 16 bit long. The above function is implemented by checking, if the memory location has exceeded this storage limit, i.e.; if the data_ptr >= 0x10008000. If the storage limit exceeds this value, then automatically, the timer interrupt is disabled and the current recording is deleted and all the previous recording are transmitted to the PC and the system is reset. The visual indication is provided by calling the overflowindicator subroutine, where the LED’s corresponding to all the functions flashes for 5 times indicating that the memory has become full.

Void overflowindicator ( )

{

int loop;

loop = 0;

While (loop <5)

{

loop ++;

GP0DAT = 0x48480000;

GP2DAT = 0x68680000;

delay(10000000);

GP0DAT = 0x48000000;

GP2DAT = 0x68000000;

delay(10000000);

}

return;

}

If the memory doesn’t become full, then the automatically, the samplecount array value is incremented to store the details of next recording, the stopflag is made high, the timer interrupt is disabled and control is transferred to the main program.

- Delete or Scrap Subroutine

The delete or scrap subroutine is included as part of the routine menu function and is used for scrapping a recorded function. Again here, the status of the ‘yesnoflag’ is initially checked so as to indicate the microcontroller to respond to the external IRQ and if the condition is satisfied only then the statements within this subroutine are executed. The scrapping function is enabled if and only if a single recording has been made. Hence, if a recording is completed and if the user is not satisfied with the recording made, it can be deleted by choosing the scrap function via the external IRQ. Once, this function is selected, the microcontroller deletes the information stored in the memory location and updates the ‘data_ptr’ and the ‘expcounter’ as shown below.

if ( expcounter>0 )

{

GP0DAT = 0x40400000

scrapflag = 1;

delay1(20000000);

if (yesnoflag != 0)

{

expcounter = expcounter-1;

data_ptr = (data_ptr) - samplecount [expcounter]*3;

GP0DAT = 0x40000000;

return;

}

scrapflag = 0;

GP0DAT = 0x40000000;

Once the subroutine is executed, the status of the corresponding flags is disabled and the control is transferred to the beginning of the menu function routine.

- Transmit subroutine

Void transmit ( )

{

new_data_ptr = (unsigned short *)0x10000000;

while(!(0x020==(COMSTA0  & 0x020))){}

COMTX = expcounter;

for (i = 1;i <= expcounter;i++)

{

tempdata = samplecount[i];

While (! (0x020== (COMSTA0 & 0x020)))

{}

COMTX = 0x0D;

While (! (0x020== (COMSTA0 & 0x020)))

{}

COMTX = ((tempdata >> 8) & 0x00FF);

While (! (0x020== (COMSTA0 & 0x020)))

{}

COMTX = (tempdata &  0x00FF);

}

for (i = 0; i<32768;i++)

{

tempdata = *new_data_ptr;

While (! (0x020== (COMSTA0 & 0x020)))

{}

COMTX = 0x0D;

While (! (0x020== (COMSTA0 & 0x020)))

{}

COMTX = ((tempdata >> 8) & 0x00FF);

While (! (0x020== (COMSTA0 & 0x020)))

{}

COMTX = (tempdata & 0x00FF);

new_data_ptr++;

}

}

The transmit function is included as part of the menu function routine. However, when the function is called, the control is transferred to the subroutine shown above.                   Basically, the data transmitted from the microcontroller has information pertaining to the number of recordings, length of each recording, carriage return, MSB data and LSB data. The carriage return is used for identifying the starting position of the data transmitted and the MSB-LSB data correspond to the data collected from all the six channels. The ‘For loops’ used in this subroutine performs this function. In the first ‘for loop’, the information pertaining to the recordings made – stored in the ‘samplecount’ array is accessed and by checking the status of the line status register (COMSTA0), the data is transmitted. The data obtained from this array is a 16 bit data and this data is split into two 8 bit (MSB-LSB) data during transmission. However, due to the format in which the data is stored by default (shown below) and  way in which the data is required to be stored (shown below), the data is rotated and to extract the actual data following the rotation, it’s logically AND’ed with OFOO.

Figure 3.3 – Data storage format

In the second ‘For loop’, the data in all the different external memory locations (32768 locations) are accessed and the same procedure (described above) is repeated to transmit the information. However, prior to data transmission, specific address to the external memory location ‘data_ptr’ is initialised and the status of COMSTA0 is checked and it is ensured that the COMTx register is ‘cleared’ to enable data transmission. Finally, once all the data recorded is transmitted, the memory is cleared and the system is reset using the command: ‘RSTSTA = 0X04’. This is included as part of the menu function routine.

The above firmware written in user friendly language is then converted into a Hex file and is serially downloaded into the flash/EEPROM of the microcontroller. This enables us to make use of SUDALS to collect and transmit data pertaining to the flexion/extension of the knee.