Information Sheet for users from clinical environment

Investigation Title

Evaluation of the user friendly nature of Strathclyde university data logging system (SUDALS) during various activities of daily living (ADL) such as walking, deep squatting, getting in and out of a chair based on the user feedback following the data collection process.

Purpose of the Study

The purpose of this study is to:

To evaluate the user-friendly nature of SUDALS during the various above-mentioned ADL by giving training to five health professionals in the use of a conventional electrogoniometry system and the new system, allow them to collect data with the systems from one healthy normal volunteer and elicit their views on the strengths and weakness of both systems. This would help us to improve the effectiveness, safety and usability of the device based on the user feedback.

Participants:

As the end users of this system would be research nurses and physiotherapists, we would like to carry out this study with a group of 5 users involving 2 research nurses, 2 physiotherapists from clinical background and 1 clinical researcher. In addition to this, the study will also involve the participation of one healthy normal subject.

Experimental Apparatus:

- Two Flexible electrogoniometers will be attached to the lateral side of the right and left knee through attachment strips which in turn are fixed to the skin using hypo allergenic adhesive tape.

- Four force sensing resistors, two on each foot – one  positioned at the heel area and the other at the half metatarsal area, will be used to determine the foot contact with the floor and mark the start and stop of events pertaining to ADL.

- Battery powered user friendly data logger system placed in a pouch and hung on the back or front of the subject depending upon their preference, with the above mentioned transducers interfaced with this system as shown in figure1.

What we will do:

A Flexible electrogoniometer is a device used for measuring the range of movement of a joint. The volunteer will have the above instruments placed on him/her as shown in figure1. Basically, each user will work in two sessions and each session will have 2 sets of data collection pertaining to test and re-test conditions. During session 1(morning) with these attachments, the volunteer would perform the various above mentioned ADL and during each of these activities, the information pertaining to the flexion-extension of the knee from flexible electrogoniometers would be collected using the commercial data collecting system by a research nurse or a physiotherapist. Similarly, in session2 (afternoon) the same user is expected to collect data from the same volunteer during these ADL using SUDALS. Prior to these sessions, complete training with a standard operating procedure would be given to the users and following these sessions, a short discussion for about 5 – 10 minutes would be conducted on a one to one basis, to know the user point of view regarding the usage of 2 different data collecting systems. In addition to this, the users would be asked to fill in a feedback questionnaire comprising of less than 10 questions. The protocol is a standard one and will be used by all the 5 users for testing the systems on the same volunteer on 5 different working days according to the convenience of both the parties involved with this study. The entire range of activities mentioned above would be carried out in the biomechanics movement analysis laboratory in the bioengineering unit.

Though the actual time involved in collecting data from the subject is only 40 minutes, due to the training, interview and different data collecting sessions; it would be greatly appreciated if the participants are able to spend about 6-7 hours for this experiment. However, sufficient breaks and refreshments would be provided to the participants on request.

Figure 1

All information would be considered confidential and will be used anonymously for research purposes only. If you have any queries regarding the project, please feel free to contact me at the address given below:

Principal Investigator:

I.Vivek Padmanaabhan,

Postgraduate Researcher,

Department of Bioengineering,

University Of Strathclyde,

106, Rottenrow,

Wolfson Centre,

Glasgow – G4 0NW.

Email: vivek.indra-mohan@strath.ac.uk