## 3.1 Registers used in the design of SUDALS firmware:

- ADCSTA: The ADCSTA is an ADC status register which represents the status of the ADC and indicates when an ADC conversion result is ready. Only when the ADC is not busy, a conversion can take place or other functionalities of the data logger can be used. In the firmware, this condition is always monitored by checking the status of the ‘busyflag’ before selecting the various functions available within the main menu. Also an A/D conversion is only performed after checking the status of the ADCSTA register. However, no specific address location is set for initialising this register.

- ADCCON: ADCCON is an ADC control register that allows the user to enable the ADC peripheral, select the mode of operation of the ADC and choose the conversion type required. It is a 16 bit register and assigning suitable address to this register depends on the application and memory mapped register (MMR) bit specification. The address assigned to this register in the firmware in the design of SUDALS is 0x00A3. Assigning this specific address location to this register enables a single ended mode, single software conversion with an ADC acquisition time of 2 clocks and a clock speed of fADC/1 to obtain 1 MSPS ADC with an external clock less than 41.78 MHz. Also, the same register is also used for powering on the ADC by assigning suitable address. By default the address – 0x620 is used for powering on the ADC. This is obtained from the sample firmware code given by the manufacturers.

- ADCCP: ADCCP is a positive channel selection register. It is an 8 bit register and similar to the ADCCON, here the address is assigned based on the MMR bit specification. The address allocated for this register in the firmware in the design of SUDALS is ADCCP = 0x01. This address corresponds to the selection of the 1st ADC channel or ADC1. Similarly the other ADC channels are also selected.

- ADCDAT: ADCDAT is an ADC data result register. Once the ADC channels are selected and configured by providing suitable address, the ADC converts the analog input and provides a 12 bit result in the ADC data register in the format as shown below in figure 3.11. The 12 bit result obtained is placed from bit 16 to bit 27 in the form of binary format.

Figure 3.1 – ADC result format

- DACxCON and DACxDAT: DACxCON is a control register similar to the ADCCON, which allows the user to enable the DAC peripherals.

- However, unlike the ADCCON, DACxCON is a 7 bit register which has MMR bit designations. Identical to DACxCON is the DACxDAT register which is 32 bit data register. The output of the DAC’s are set in the range of 0 to Vref (2.5 V) and the outputs from the data are obtained via the DACxDAT register. In the design of the firmware for SUDALS, the DAC registers are used for indicating the function selection time out functionality and for zeroing the transducers (as explained in the hardware section).

- REFCON:  The ADUC7026 provides an on-chip band-gap reference voltage of 2.5 V, which can be used for the ADC’s and DAC’s. This functionality is utilised by designating the address 0x01 to the REFCON by enabling the corresponding bits in the MMR REFCON.

- RSTSTA: The software reset functionality is included in the firmware by enabling the corresponding bit in the 8 bit MMR RSTSTA register. This results in a register address – 0x4.

- ISR & Timer Registers:  IRQEN and IRQSTA are the two important ISR registers used in providing the required ISR to the MCU in the design of SUDALS. Both these registers are 32 bit interrupt registers. IRQEN is an interrupt register which provides the value of the current enable mask. When the bit value is set to 1, the source request is enabled to create an IRQ exception. Alternatively, when the bit value is set to 0, the source request is not created.

IRQSTA is a read only register which provides the current enabled IRQ source status. When the register value is set to 1, the source generates an active IRQ request to the ARM7 core and this function is implemented via software in a common interrupt handler routine. Reviewing the sample ISR firmware codes, revealed that from an application point of view, the manufacturers have defined the IRQEN register as a summation of an external interrupt request bit and a timer bit.

IRQEN = XIRQO_BIT + GP_TIMER_BIT.

This allows us to use this register as a Timer register and as an IRQ register for checking the ISR conditions. In the design of SUDALS, the IRQEN register is used along with the IRQSTA to check two different conditions due to its dual functionality. It is used to initiate a recording process after the MCU receives the external interrupt trigger signal and then initiates the A/D conversion based on the timer conditions. The manufacturer documentation reveals that, the timer 1/ timer bit has a capture register which can be triggered by a selected IRQ source and timer 1 can be used to start A/D conversion. Also, the timer bit enables the timer interrupt and routes the timer to the IRQ handler when the timer overflows (which is explained in detail in the following paragraphs).

Basically, the timer used here is a count down timer and when the timer is enabled, the timer starts running and start the count down. The running time of the timer is set to 20 ms corresponding to 50 Hz sampling time. Hence, after the 20th ms, the timer starts to overflow and causes the timer interrupt to occur. This in turn is routed to the IRQ handler and every time the timer creates an interrupt, the data read previously, is sampled (at 50 Hz) and converted to digital values. Again after 20 ms, next sampling takes place. However, every time for the timer to create an interrupt at that particular instant, the timer IRQ has to be cleared. The timers used for the above applications are; TxLD, TxCON and TxCLR.

  - TxLD is 16 bit load register. The timer counter (mentioned above) decrements or increments from the value stored in the load register until zero/full scale and then starts again at the value stored in the load register. The timer interval is calculated using a relation given by the manufacturers:

Timer interval = [(TxLD) * Prescaler] / source clock.

In the design of SUDALS, the TxLD value is calculated based on the desired sampling frequency (50 Hz). Substituting this in the well known relation, frequency (F) = 1/ time (T), gives us the T = 20 ms, which is the timer interval corresponding to 50 Hz. From the sample firmware code given by the manufacturers, in a similar application the TxLD value for 50 ms was calculated to be 20,000. Hence, the TxLD value corresponding to 20 ms is equivalent to 8000. This assigns an address: 0x8000 to the TxLD or T1LD register.

  - T1CON is a 32 bit control register which is enabled by selecting the desired MMR BIT specifications. The MMR BIT specification allows one to enable the Timer 1 bit by selecting the source clock, the required prescaling factor, the required format and mode of operation of Timer1. The source clock can be either a 32 KHz external crystal or the core clock frequency or the internal 32 KHz oscillator (manufacturer documentation). The clock source can be scaled by a factor of 1, 16 or 256. In the design of SUDALS, the source clock is a core clock scaled by a factor of 16 operated in a periodic mode and these results in a register address 0x04 when the timer is initially disabled and 0xC4 when the timer is enabled.

  - Further, to clear the Timer 1 interrupt at a particular point before the initiation of the sampling every time, an 8 bit timer clear register is enabled (T1CLR).

- External Memory registers: Three external memory registers are used in the                  design of SUDALS. They are XMCFG, XM0CON and XM0PAR.

  - XMCFG is the external memory configuration register. The register value is set to 1 to enable the external memory registers. Along with this register, the corresponding GPIO port pins are also enabled via the GPxCON MMR BIT specifications.

  - XMxCON register controls each memory region and assigning suitable address to these registers allows one to enable or disable a memory region and control the data bus width of the memory region. Enabling the memory region and selecting a 16 bit data bus results in, XM0CON = 0x3.

  - XM0PAR register defines the protocol used for accessing the external memory for each memory region and by default the address of this register is: XM0PAR = 0x70FF and the same address is used in the firmware of SUDALS, which in turn gives us 16 clock cycles corresponding to read wait and write wait states.

- Communication registers:    The following communication registers are used for interfacing the UART with the wireless RS232 communication port and transmitting the data wireless. COMTx is 8 bit transmit register and COMDIV0 is a low byte divisor latch register. Both these registers share the same address locations. The COMTx can be accessed only when the bit 7 in the line control register – COMCON0 register is cleared and COMDIV0 can be assessed when BIT 7 of COMCON0 is set. COMSTA0 is a line status register and the bit 5 of this register is automatically set to 1 if the COMTx register becomes empty. Only if the transmitter register is cleared then the data can be transmitted. Hence, this condition is always checked prior to data transmission, by comparing the status of the COMSTA0 with the address 0x020, which is the equivalent address obtained when the bit 5 of this register is set to 1. From the sample firmware codes given by the manufacturers, implementation of the COMCON and COMDIV registers for transmitting the data at a baud rate of 19,200 is applied here in the design of SUDALS. COMCON0 is an 8 bit register and the address assigned to this register is 0x080 and 0x007 to enable and disable access to COMDIV0 and COMDIV1 registers and enables us to set the word length to 8 bits. Further, from the manufacturers’ documentation, the address corresponding to a specific baud rate (19200) is obtained and is assigned to the COMDIV0 register – 0x044 and from the sample firmware code, the address corresponding to the COMDIV1 (0x000) is obtained. This allows us to transmit 8 bit data at a baud rate of 19,200 with 1 stop bit with no parity transmission check.

- GPIO Registers: ADUC7026 provides 40 general purpose bidirectional input output pins. All the input output pins can support an input voltage of 5V and each input output pin has multiple functions. Hence, careful selection of the pins depending on the application is mandatory. In the design of SUDALS various GPxCON and GPxDAT registers are used. GPxCON registers are port x control registers and are used for selecting the functions of each pin of port x. On the other hand, GPxDAT registers configure the direction of the GPIO pins of port x, sets the output value for the pins configured as output and stores the input value for the pins configured as input. Suitable addresses are assigned to these registers to enable the external memory location and communication ports as mentioned earlier. These addresses are obtained from the sample firmware codes given by the manufacturers.

GP1CON = 0x011 is used for setting up the transmission and receiver pins and GP0CON = 0x220, GP2CON = 0x00022220, GP3CON = 0x22222222, and GP4CON = 0x22222222 are used for setting the external memory mode for bit high enable and bit low enable.

In addition to this, various input output ports used in this application are P0.4, P0.3, P0.6, P2.5, P2.6 and P2.7. The port P0.4 is used for interfacing the output of the remote control circuit to the microcontroller for providing the desired trigger (external IRQ) and the pins 2.7, 0.3, 0.6, 2.5 and 2.6 are connected to LED’s and are used for indicating the functions like zeroing, recording, scrapping, transmitting and reset. These LED’s are switched on/off by enabling and disabling suitable bits in the GPxDAT registers corresponding to these ports.

With the use of all these registers explained above, the logic for the operation of SUDALS is designed as explained below.