| GREENCODE Issue No: |  |
| --- | --- |

Document No. 3.1.3

3. Environmental Management Manual

1. Management Procedures

3. Identification of Significant Effects

Revision 1.1

Management Procedure

Guidance for Providers of Healthcare on: Identification of Significant Environmental Effects

© Crown Copyright 1995.

First Published 1995.

Prepared by the NHS in Scotland & HSS Northern Ireland Estates Environment Forum.

Printed by the Safety & Environmental Management Unit, University of Strathclyde.

All Rights reserved. No part of this publication may be reproduced in any form or by any means without the prior permission of the Scottish Office and Department of Health and Personal Social Services Northern Ireland.

While every care has been taken to ensure the accuracy of this document, the developers can accept no responsibility for errors or omissions. The person using this document must satisfy him/herself as to its accuracy and relevance having regard to the circumstances of its use.

| Principal Authors | P P Yaneske & K McDonach |
| --- | --- |
| Issue Date | 13 July 1995 |

1.	Summary of Procedure

Environmental effects require to be given a rating to show the level of significance that is placed upon them by management. This process also helps to determine those issues which should be attended to first and further helps with the generation of initial environmental management objectives.

This procedure provides guidance regarding the identification of significant environmental effects. It outlines the personnel concerned with its implementation, the work instructions to be followed and the methods of recording and verification on its completion.

2.	Purpose of Procedure

The purpose of this procedure is to aid management evaluate the level of significance given to each environmental effect previously identified using GREENCODE document 3.1.1.

3.	Implementation of Procedure

This procedure shall be implemented by a Senior Officer, supported by an environmental management team.

4.	Verification

Verification of particular elements within the GREENCODE Environmental Management System will be required in the future. Verification of this procedure will be monitored by the completion of document 3.1.3.(SF/01). The following work instructions indicate how this document can be generated manually or from within the GREENCODE database software.

5. 	Records

Certain records require to be maintained within the GREENCODE Environmental Management System to verify past activities and prove compliance with the Trust environmental policy and objectives. To assist with these requirements the completed document 3.1.3.(SF/01) associated with this procedure must be retained and made available in a readable manner to any auditors in the future.

When this procedure is completed for the first time (as part of your Preparatory Review) the record should be stored in section 7 of the GREENCODE folder.

6.	Selected References

1	BS 7750: Specification for Environmental management systems. BSi, 1994.

ISBN 0-580-22829-0.

7.	Appendices

None

N.B.	This procedure follows directly GREENCODE documents 3.1.1 'Identification of Environmental Effects' and 3.1.2 'Identification of Applicable Legislation.

8.	Work Instructions

(1)	From within the GREENCODE DATABASE select 'open' from the 'Review' menu.  From the 'List of reviews' presented double click on the one of your choice. You will then be presented with the GREENCODE review screen.

(2)	At the bottom right hand corner of the screen you will see a button entitled 'Print Significant Effects Worksheet'. Selection of this button will generate GREENCODE document 3.1.3.(SF/01). If you have previously identified the applicable legislation using GREENCODE document 3.1.2, the worksheet will be partially completed when printed.

(3)	The printout will produce a worksheet of the kind outlined on page 4 of this procedure. The first column will be filled with the effects you selected using GREENCODE document 3.1.1. If you have previously identified the applicable legislation using GREENCODE document 3.1.2, the second and last column of the worksheet will be automatically completed such that any environmental effect identified as having legislation associated with it will be awarded the maximum score of 100 (one hundred) in the last column.

(4)	The rest of the worksheet is now to be completed using the following criteria.

·	Column 3 - If your site or Trust has a policy relating to an effect in column 1, then the maximum score of 100 is entered in the last column against the effect.

·	Column 4 - If you subscribe to a code of practice relating to an effect in column 1, then the maximum score of 100 is also entered in the last column against the effect.

If any of Columns 2, 3 or 4 are identified as applicable then all other Columns are to be ignored and a score of 100 (one hundred) entered in the last column.

(5)	If Columns 2, 3 and 4 are not applicable, then the remaining four Columns are to be scored subjectively between 1 and 25 giving a possible maximum score of 100 for the effect.

·	Column 5 - make an estimate as to the impact that the particular environmental effect has on HUMAN HEALTH.

·	Column 6 - make an estimate as to the strength of SCIENTIFIC EVIDENCE about the impact of the particular environmental effect.

·	Column 7 - make an estimate as to the level of PUBLIC PERCEPTION relating to the impact of the particular environmental effect.

·	Column 8 - make an estimate as to the importance the PURCHASER of your service places on the impact of the particular environmental effect.

(6)	The scores entered in the last column now identify the level of significance associated with each of the listed environmental effects.

(7)	The completed GREENCODE document 3.1.3.(SF/01) should now be retained as a record of your decisions. If this is the first time this has been carried out for a particular site, then it should be recorded as part of the site 'Preparatory Review' and should be stored in Section 7 of your GREENCODE manual.

(8)	The management team should review the significance of environmental effects regularly (e.g. once per year).

End of Procedure

Significant Environmental Effects Worksheet

| Environmental Effect | Legal Compliance | Trust Policy | Code of Practice | Effect on Health | Scientific Evidence | Public Perception | Purchaser Criteria | Significance Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Boiler Flue Emissions | X |  |  |  |  |  |  | 100 |
| Flushing of Water Systems |  | X |  |  |  |  |  | 100 |
| Autoclave/Steriliser Waste |  |  | X |  |  |  |  | 100 |
| Dust |  |  |  | 10 | 15 | 20 | 20 | 65 |